|  | EUROPEAN COMMISSION DIRECTORATE-GENERAL III INDUSTRY Regulatory policy and standardisation, electronic commerce, telematics networks, environment |
| --- | --- |

Brussels, 6 July 1999

DGIII/D3 GE/si D(99)

M134 EN

AMENDMENT TO :

HORIZONTAL COMPLEMENT TO THE MANDATES

TO CEN/CENELEC

CONCERNING THE EXECUTION OF STANDARDISATION  WORK FOR

THE EVALUATION OF CONSTRUCTION PRODUCTS IN RESPECT OF THEIR RESISTANCE TO FIRE

(approved by the SCC as document Construct 96/207 Rev.2)

The above mentioned mandate is hereby amended as follows :

1. AMENDMENT TO ANNEX 1

The following products are added under the sub-heading “Products and systems for protecting elements or parts of the works” :

“Wall and ceiling coverings”

2. AMENDMENT TO ANNEX 2

2.1 Section II.J

In the first paragraph of section II.J, the sentence which now reads “Products and elements where the external surface or surfaces of the chimney are within a building must satisfy an insulation requirement, defined as being a maximum temperature rise not exceeding 140°C at any point on the surface.” is replaced by the following :

“Products and elements where the external surface or surfaces of the chimney are within or adjacent to a building must satisfy an insulation requirement defined as being a maximum temperature of adjacent materials not exceeding 100oC when related to an ambient temperature of 20oC.

Any distance to combustible material necessary to achieve this requirement shall be declared. This value shall not exceed the distance required to satisfy the criteria for normal operating conditions.

The G classification will be followed by the designation of the necessary distance.”

2.2 Section II.K

The following text is added to section II.K :

“The use of the X classification for the fire protection ability of wall and ceiling coverings is explained in section IV of this annex.”

2.3 Section III.10

The following text is added to section III.10 :

“The use of the X classification for the fire protection ability of wall and ceiling coverings is explained in section IV of this annex.”

2.4 Section IV.C

The following text is added to section IV.C :

“Classification of wall and ceiling coverings (for fire protection ability)

Wall and ceiling coverings may be required to provide protection to combustible materials and cavities behind the visible surface.

- Exposure/action :  “standard temperature/time” curve.

- Performance criteria : shall be appropriate to the determination of the fire protection ability of the covering .

- Classification : X, indicating that the performance criteria have been passed.”

3. OTHER

CEN/ CENELEC shall present the Commission with a detailed proposal for the work programme, at the latest three months after approval of this mandate by the 98/34 Committee.

The European standards shall be prepared in accordance with the provisions of the original mandate.

CEN are requested to determine an appropriate symbol for this classification.

CEN are requested to standardise the Nordtest method NT FIRE 003 “Coverings : Fire protection ability” and to take account of existing regulations in Europe in elaborating the test conditions and performance criteria for this classification.