Results obtained from the EGF consultation on the TC 129 proposal concerning the reaction to fire classification of certain glass products

- The EGF experts stressed the need to maintain the agreed European assessment method on Reaction to Fire without exceptions. In their opinion the Reaction to fire classification of glass products ought to follow the same rules as all other building products.

- It was felt that it would not be acceptable to give to a product a classification when it is been shown NOT to achieve that classification. This risks creating a not acceptable precedent.

- Most EGF members reminded that the application of the appeal procedure foreseen in the Guidance Paper G is an available option to the industry. The said procedure can be followed in cases where the SBI test is believed to result in an unrepresentative classification for specific product families.

- Concerning requirements in building regulations one should examine two different cases:

A) In those Member States which do not regulate on Reaction to Fire, the Reaction to Fire performance of glass products does not pose any problem because the declaration of their performance concerning this ess. characteristic is not a legal precondition in order to market and use these products.

B) For those Member States which regulate on Reaction to Fire, most EGF members considered that the regulators of each Member State, should be invited to take due account of the performance obtained by well known glass products under the European assessment and classification method and decide, if he/she considers necessary and appropriate, on possible adjustments in the national building provisions and provide specific guidance. In other words, the fire regulators may decide at national level for specific end use applications (taking into account e.g. building type, location, etc.) on the performance which the specific glass products should have and then address this issue through national relaxation of building regulation requirements and/or inclusion of compensatory fire protection measures and/or application of fire safety engineering.

- Concerning the use of NPD option, it needs to be stressed that the manufacturer is always allowed to declare that the performance of his product concerning the ess. characteristic “Reaction to Fire” has not been determined (NPD = No Performance Determined) if he prefers so and if the product standard does not oblige the manufacturer to declare the performance of the products for this characteristic. To this end the product does not need to be tested against reaction to fire.

- The EC services would like to underline on this occasion that the achievement of a common European technical language on the assessment of the performance of construction products has indeed been one of the objectives of the CPD/CPR. However this does not mean that the conditions on the use of construction products would necessarily have to be the same in all EU Member States. The Member States authorities are thus fully entitled to take into account the different levels of protection and of different geographic and climatic conditions which prevail in the Member States.