|  | EUROPEAN COMMISSION ENTERPRISE AND INDUSTRY DIRECTORATE-GENERAL Chemicals and construction Construction |
| --- | --- |

Brussels, May 2009

ENTR/G5/MF D(2005)

DS 041/051 rev.9

Indicative List of Regulated Dangerous Substances possibly
associated with Construction Products under the CPD

1.	Background/Purpose of this List

Construction products could emit or contain substances that have been defined as “dangerous substances” under Commission Directives and national regulations. While the emission (or the content – whenever this is the most practicable solution or required by regulations) of any such substances from construction products (or in construction products) must be below all relevant European and national threshold values, manufacturers and authorities profit from a transparent performance information in the CE marking of the product.

To provide more guidance on regulatory requirements on EU and Member State level to CEN TC 351, product TCs and EOTA WG, a three-step-approach has been chosen:

- A general overview on EU Directives and Regulations as well as on national regulations on dangerous substances in or emitting from construction products in the database on dangerous substances. The database should be seen as the most complete approach possible of providing information to specification writers what regulations might be relevant for specific products.

- To provide a better indication on main priorities, this indicative list indicates on which substances and parameters, which CEN TC 351 should focus, when assessing the availability of test methods and the need for developing harmonised test methods (as foreseen in Mandate M/366), and should also provide more detailed guidance for CEN product TCs and ETOA WG.

- While the first two steps should raise awareness on potential regulatory requirements for specific products and help TCs and WGs to identify requirements relevant for their products or product groups, only the last step should provide clear and comprehensive guidance for each product or product group: By evaluating all Commission mandates under the CPD, the Commission, its Expert Group on Dangerous Substances in the field of Construction Products (EGDS) and CEN product TCs/EOTA WGs will define together which specific substances, materials and parameters should be addressed in the mandate and should be covered in all relevant harmonised European product standards.

Therefore, this indicative list has to be seen as a tool to facilitate the work of TC 351 but also of product TCs and EOTA WGs to identify relevant substances, materials and parameters, still open for expert input from specification writers for product specific and precise amendments of specific product mandates.

This updated document substitutes the previous version of the indicative list (DS 041/051 rev.8) of 31st October 2006.

The list is based on the database on legislation on dangerous substances relevant for construction products developed by the Commission in cooperation with Member States. This database should also be used by CEN TC 351 in combination with the expertise of the product TCs and EOTA WGs.

On the EU level, in particular the following regulations and directives have to be taken into consideration:

- REGULATION (EC) No 1907/2006 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 18 December 2006 concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH), establishing a European Chemicals Agency, amending Directive 1999/45/EC and repealing Council Regulation (EEC) No 793/93 and Commission Regulation (EC) No 1488/94 as well as Council Directive 76/769/EEC and Commission Directives 91/155/EEC, 93/67/EEC, 93/105/EC and 2000/21/EC

  - ANNEX XVII: RESTRICTIONS ON THE MANUFACTURE, PLACING ON THE MARKET AND USE OF CERTAIN DANGEROUS SUBSTANCES, PREPARATIONS AND ARTICLES

- REGULATION (EC) No 1272/2008 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 16 December 2008 on classification, labelling and packaging of substances and mixtures, amending and repealing Directives 67/548/EEC and 1999/45/EC, and amending Regulation (EC) No 1907/2006

- Directive 98/8/EC of the European Parliament and of the Council on the placing on the market of biocidal products

- DECISION No 2455/2001/EC OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 20 November 2001 establishing the list of priority substances in the field of water policy and amending Directive 2000/60/EC

- Directive 2006/118/EC of the European Parliament and of the Council of 12 December 2006 on the protection of groundwater against pollution and deterioration

- Council Directive 80/68/EEC of 17 December 1979 on the protection of groundwater against pollution by certain dangerous substances

Note: The user of the document should keep in mind that, while the list seems long, not all substances have to be tested by the producer nor has CEN to develop new test methods for every substance or parameter. Firstly, test methods for a large number of substances/materials already exist and have only to be harmonised. Secondly, substances can be arranged in groups and analysed using the same method. Substances not used may be closed out in the product description.

# 2.	Selection of Substances: Guidance from the Expert Group

The main focus of current work on dangerous substances in CEN/TC 351 is the finalisation of test methods for the different release scenarios. The choice of substances to be tested will follow on from this in the next step.

The attached lists are based on the Mandate M/366, on European Regulations (see above) and related national legislation. Further substances, included in other directives and national legislation, may be discussed in the Expert Group after the database on regulations with regard to dangerous substances has been updated and subsequently evaluated in detail. The attached lists should be considered as an updated orientation for the work programme of CEN/TC 351. The lists are open and can be updated again if needed.

The lists presented here are based on the assumption of a specified quality of raw materials and specified product formulations, or on ‘good housekeeping’. The listed substances / parameters were chosen because they:

- are regulated on EU or national level

- are components of construction products,

- are used for manufacturing construction products (included in raw materials)

- could be emitted or released from construction products into indoor air

- could be released from construction products into soil, surface water and ground water

- are used in regulations to characterise products or belong to key parameters for a group of substances or an environmental impact (such as pH, conductivity, DOC etc.)

The indicative list is subdivided into the following subchapters:

Annec A: List A “Soil and water”

A-1. Regulated dangerous substances in main pollutant categories of Directive 2000/60/EC (Water Framework Directive)

A-2. Further regulated substances and parameters

A-3. Other substances deemed relevant: PBT substances

Annex B: List B “Indoor air”

B-1 Regulated dangerous substances and further associated parameters: overview

B-2 Regulated volatile organic compounds (VVOC, VOC, SVOC)

B-3 Carcinogenic VOC

Annex C: List of consulted notifications

# 3.	Use of the Indicative List

In the first two columns chemicals are described as groups and single substances respectively. For single substances the corresponding CAS Number is mentioned in the third column. The last column is reserved for regulatory observations (regulatory classifications and references to European or national provisions). For the meaning of the risk phrases given in this column Annex II and III of the COMMISSION DIRECTIVE 2001/59/EC adapting to technical progress for the 28th time Council Directive 67/548/EEC on the approximation of the laws, regulations and administrative provisions relating to the classification, packaging and labelling of dangerous substances may be consulted. CEN/TC 351 is asked to identify, if test methods for the listed substances and parameters are available, and make recommendations for the choice of appropriate methods to be used in hEN, ETAGs and CUAPs.

***

Annex A

List A “Soil/Water”

Explanation of used colours:

blue = regulation for content

yellow = regulation for release

purple = regulation for content and release

grey = regulation for environmental quality

green = PBT-substance (persistent, bioaccumulative, toxic; see also table A-3)

red = CMR-substance (carcinogenic, mutagenic or toxic for reproduction) of EU categories 1 and 2

A-1.	Regulated dangerous substances

The structure of the table follows the main pollutant groups defined in Annex VIII of DIRECTIVE 2000/60/EC OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 23 October 2000 establishing a framework for Community action in the field of water policy. Further regulated anions, sum parameters and indicator parameters are listed in Table A-2, PBT substances in table A-3.

Tab. A.1 Regulated dangerous substances in context of release into soil, surface water and groundwater

| Substance group | Substance or substance subgroup |  | Chemical Abstract Number (CAS) | Regulations / Exemplary Notifications / Classifications |
| --- | --- | --- | --- | --- |
| Organohalogen compounds |  |  |  |  |
| Chlorobenzenes | Trichlorobenzenes |  | 12002-48-1 | Directive 2000/60/EC 1,2,4-trichlorobenzene: Xn, R22; Xi, R38; N, R50/53 |
|  | 1,2,4-trichlorobenzene |  | 120-82-1 |  |
|  |  Chlorobenzenes |  | [108-90-7] [95-50-1] [541-73-1] [106-46-7] [87-61-6] [108-70-3] [634-66-2] [634-90-2] [95-94-3] [608-93-5] | 2006-90-D, 2006-557-NL - Chlorobenzene: R10; Xn, R20; N; R51/53 - 1,2-dichlorobenzene: Xn, R22; Xi, R36/37/38; N, R50/53 - 1,3-dichlorobenzene: Xn, R22; N; R51/53 - 1,4-dichlorobenzene: Xi, R36; Carc.Cat.3, R40; N; R50/53 - 1,2,3-trichlorobenzene: Xn, R22; N, R50/53 - 1,3,5-trichlorobenzene: Xn - 1,2,3,4-tetrachlorobenzene: Xn - 1,2,3,5-tetrachlorobenzene: Xn - 1,2,4,5-tetrachlorobenzene: Xn - Pentachlorobenzene: F, R11; Xn, R22; N, R50/53 |
|  | Hexachlorobenzene |  | [118-74-1] | 2006-90-D Carc.Cat.2, R45; T, R48/25; N, R50/53 |
|  Chlorinated phenols |  |  | [95-57-8] [108-43-0] [106-48-9] [576-24-9] [120-83-2] [583-78-8] [87-65-0] [95-77-2] [591-35-5] [15950-66-0] [933-78-8] [933-75-5] [95-95-4] [609-19-8] [88-06-2] [4901-51-3] [935-95-5] [58-90-2] [87-86-5] | 2006-90-D 2006-557-NL - 2-chlorophenol, 3-chlorophenol, 4-chlorophenol: Xn, R20/21/22; N, R51/53 - 2,3-dichlorophenol, 2,5-dichlorophenol, 3,4-dichlorophenol, 3,5-dichlorphenol, 2,3,4-trichlorophenol, 2,3,5-trichlorophenol, 2,3,6-trichlorophenol, 3,4,5-trichlorophenol: Xn - 2,4-dichlorophenol: T, R24; Xn, R22; C, R34; N, R51/53 - 2,6-dichlorophenol: Xi - 2,4,5-trichlorophenol: Xn, R22; Xi, R36/38; N, R50/53 - 2,4,6-trichlorophenol: Carc.Cat.3, R40; Xn, R22; Xi, R36/38; N, R50/53 - 2,3,4,5-Tetrachlorophenol: T, R25; R37/38 - 2,3,5,6-tetrachlorophenol: T, R25; R36/38 - 2,3,4,6-tetrachlorophenol: T, R25; Xi, R36/38; N, R50/53 - Pentachlorophenol: Carc.Cat.3, R40; T+, R26; T, R24/25; Xi, R36/37/38; N, R50/53 |
| Volatile halogenated hydrocarbons |  | Halogenated C1- and C2- hydrocarbons, including trihalogenated methane | [74-87-3] [74-83-9] [74-95-3] [75-34-3] [75-00-3] [106-93-4] [75-25-2] | 2006-90-D - Methyl chloride: F+, R12; Carc.Cat.3, R40; Xn, R48/20 - Bromomethane: Muta.Cat.3, R68; T, R23/25; Xn, R48/20; Xi, R36/37/38; N, R50; N, R59 - Dibromomethane: Xn, R20; R52/53 - 1,1-dichloroethane: F, R11; Xn, R22; Xi, R36/37; R52/53 - Chloroethane: F+, R12; Carc.Cat.3, R40; R52/53 - 1,2-dibromoethane: Carc.Cat.2, R45; T, R23/24/25; Xi, R36/37/38; N, R51/53 - Tribromomethane: T, R23; Xi, R36/38; N, R51/53 |
|  |  | Methylene chloride | 75-09-2 | Directive 2000/60/EC, 2006-557-NL, 2006-90-D Carc.Cat.3, R40 |
|  |  | Ethylene dichloride | 107-06-2 | Directive 2000/60/EC, 2006-90-D F, R11; Carc.Cat.2, R45; Xn, R22; Xi, R36/37/38 |
|  |  | Chloroform | 67-66-3 | Directive 2000/60/EC, 2006-557-NL, 2006-90-D Xn, R22-48/20/22; Xi, R38; Carc.Cat.3, R40 |
|  |  | Vinyl chloride | 75-01-4 | 2006-90-D, 2006-557-NL F+, R12; Carc.Cat.1, R45 |
| C10-13-chloroalkanes (short chain chlorinated paraffins) |  |  | 85535-84-8 | Directive 2000/60/EC Commission Decision 2004/1/EC Carc.Cat.3, R40; N, R50/53 |
| C14-17-chloroalkanes (medium-chained chlorinated paraffins) |  |  | 85535-85-9 | 2007-9016-N |
| Polyhalogenated dibenzo-p-dioxins and polyhalogenated dibenzofuranes |  |  | Not applicable | 1993/141/D |
| Polychlorinated biphenyls (PCB) |  |  | 1336-36-3 | Directive 76/769/EEC, 2006-90-D, 2005-735-FIN, 2006-557-NL Regulations based on the sum of seven PCB congeners (28, 52, 101, 118, 138, 153 and 180) or on the sum of 6 PCB congeners (28, 52, 101, 138, 153, 180). R33; N, R50/53 |
| Tetrabromobisphenol A |  |  | 79-94-7 | 2007-9016-N N, R 50/53 |
| Hexabromocyclododecane |  |  | 25637-99-4, 3194-55-6 | 2007-9016-N Xi, R 43 |
| Partially and fully fluorinated hydrocarbons, perfluorocarbons (HFCs, FCs, PFCs) |  |  | Not applicable | 2001-121-DK, 2002-37-A |
| Brominated diphenylethers |  | Pentabromo-diphenylether | 32534-81-9 | Directive 76/769/EEC Xn, R48/21/22; R64; N, R50/53 |
|  |  | Octabromo-diphenylether | 32536-52-0 | Directive 76/769/EEC Repr.Cat.2, R61; Repr.Cat.3, R62 |
|  |  | Decabromo-diphenylether | 1163-19-5 | 2005-255-D, 2005-9020-N R53 |
|  |  | Brominated diphenylethers as a group | Not applicable | Directive 2000/60/EC |
| Adsorbable / Extractable organic halides |  | AOX (sum parameter for halogenated substances) | Not applicable | 2004-71-D |
|  |  | EOX (sum parameter for halogenated substances) | Not applicable | 2004-71-D, 2006-557-NL |
| Organophosphorus compounds |  |  |  |  |
| Organophosphorus compounds as a group |  |  | Not applicable | Directive 2000/60/EC |
| Organotin Compounds |  |  |  |  |
| Tributyltin compounds |  |  | Not applicable | Directive 2000/60/EC, 2007-9016-N T, R25-48/23/25; Xn, R21; Xi, R36/38; N, R50/53 Bis(tributyltin)oxide (TBTO): PBT |
| Triphenyltin compounds |  |  | Not applicable | 2007-9016-N T, R23/24/25; N, R50/53 |
| Substances and preparations, or the breakdown products of such, which have been proved to possess carcinogenic or mutagenic properties or properties which may affect steroidogenic, thyroid, reproduction or other endocrine-related functions in or via the aquatic environment |  |  |  |  |
| Volatile aromatic hydrocarbons |  |  Alkylated benzenes benzene, toluene, ethylbenzene, o-xylene, m-yylene p-xylene | 71-43-2] [108-88-3] [100-41-4] [95-47-6] [108-38-3] [106-42-3] | 2006-90-D, 2006-557-NL - Benzene: F, R11; Carc.Cat.1, R45; Muta.Cat.2, R46; T, R48/23/24/25; Xn, R65; Xi, R36/38 - Toluene: F, R11; Repr.Cat.3, R63; Xn, R48/20-65; Xi, R38; R67 - Ethylbenzene: F, R11; Xn, R20 - Xylenes: R10; Xn, R20/21; Xi, R38 |
|  |  | Benzene | 71-43-2 | Directive 2000/60/EC 2006-90-D, 2006-557-NL F, R11; Carc.Cat.1, R45; Muta.Cat.2, R46; T, R48/23/24/25; Xn, R65; Xi, R36/38 |
| Epichlorohydrine |  |  | 106-89-8 | 2006-90-D R10; Carc.Cat.2, R45; T, R23/24/25; C, R34; R43. Evidence of endocrine disruptive effects. |
| Acryl amide |  |  | 79-06-1 | 1998-9024-N Carc.Cat.2, R45; Muta.Cat.2, R46; Repr.Cat.3, R62; T, R25-48/23/24/25; Xn, R20/21; Xi, R36/38; R43 |
| Methyloacrylamide |  |  | 924-42-5 | 1998-9024-N Xn; Degrades to acryl amide in the environment. |
| Asbestos |  | - Actinolite | 77536-66-4 | Directive 76/769/EEC. 2004-294-NL, 2004-410-NL. 2006-557-NL Carc.Cat.1, R45; T, R48 |
|  |  | Amosite | 12172-73-5 |  |
|  |  | Anthophyllite | 77536-67-5 |  |
|  |  | Chrysotile | 12001-29-5 |  |
|  |  | Crocidolite | 12001-28-4 |  |
|  |  | Tremolite | 77536-68-6 |  |
| Persistent hydrocarbons and persistent and bioaccumulable organic toxic compounds |  |  |  |  |
| Polyaromatic hydrocarbons (PAH) |  | PAH | Not applicable | Directive 2000/60/EC. Directive 76/769/EEC (tar oils). 2006-90-D, 2004-71-D, 2006-557-NL, 2005-735-FIN, 1999-263-A. 2007-385-A The individual substances which are considered differ in the different member states. |
|  |  | Anthracene | 120-12-7 | Directive 2000/60/EC, 2006-90-D, 2006-557-NL Xn, R36/37/38, R42/43 |
|  |  | Benzo[a]pyrene | 50-32-8 | Directive 2000/60/EC, 2006-90-D, 2006-557-NL Carc.Cat.2, R45; Muta.Cat.2, R46; Repr.Cat.2, R60-61; R43; N, R50/53 |
|  |  | Dibenz[a,h]anthracene | 53-70-3 | 2006-90-D Carc.Cat.2, R45; N, R50/53 |
|  |  | benz(a)anthracene | 56-55-3 | 2006-557-NL Carc.Cat.2, R45; N, R50/53 |
|  |  | Benz(e)acephenanthrylene | 205-99-2 | Directive 2000/60/EC, 2006-90-D Carc.Cat.2, R45; N, R50/53 |
|  |  | Benzo[k]-fluoranthene | 207-08-9 | Directive 2000/60/EC, 2006-90-D, 2006-557-NL Carc.Cat.2, R45; N, R50/53 |
|  |  | Benzo[ghi]perylene | 191-24-2 | Directive 2000/60/EC, 2006-90-D, 2006-557-NL Xn, R40 |
|  |  | Fluoranthene | 206-44-0 | Directive 2000/60/EC, 2006-90-D, 2006-557-NL Xn, R22 |
|  |  | Indeno(123-cd)pyrene | 193-39-5 | Directive 2000/60/EC, 2006-90-D, 2006-557-NL |
|  |  |  Naphthalene and methylnaphthalenes | Not applicable | 2006-90-D |
|  |  | Naphthalene | 91-20-3 | Directive 2000/60/EC, 2006-557-NL Carc.Cat.3, R40; Xn, R22; N, R50/53 |
|  |  | Phenanthrene | 85-01-8 | 2006-557-NL N, R 50/53 |
|  |  | Chrysene | 218-01-9 | 2006-557-NL Carc.Cat.2, R45; Muta.Cat.3, R68; N, R50/53 |
| Phenols |  | Nonylphenols | 25154-52-3 | Directive 2000/60/EC, 2006-90-D Repr.Cat.3, R62; Repr.Cat.3, R63; Xn, R22; C, R34; N, R50/53 Nonylphenolethoxylate: Evidence of endocrine disruptive effects. |
|  |  | 4,4'-Isopropylidenediphenol | 80-05-7 | 2007-9016-N Repr.Cat.3, R62; Xi, R37-41, R43 |
|  |  | Phenol | 108-95-2 | 2006-557-NL Muta.Cat.3, R68; T, R23/24/25; Xn, R48/20/21/22; C, R34 |
|  |  | Phenol index as a sum parameter | Not applicable | 2004-71-D, 2006-90-D Phenols (C9-11): Carc.Cat.2, R45 |
| Hydrocarbons (petrolium) |  |  | Not applicable | Directive 80/68/EEC (List 1) 2006-90-D, 2006-557-NL, 1999-263-A, 2007-385-A, 2007-653-A |
| Phthalates |  | Di-iso-octyl phtalate; Bis-(2-ethylhexyl)-phthalate (DEHP) | 117-81-7 | Directive 2000/60/EC Repr.Cat.2; R60-61 |
|  |  | Phthalates (total) | Not applicable | 2006-557-NL |
| Cyanides |  |  |  |  |
| Cyanides |  |  | Not applicable | Directive 80/68/EEC (List 1), Directive 2000/60/EC, 2006-557-NL, 2004-71-D, 2006-90-D T+, R26/27/28; R32; N. R50/53 |
| Metals and their compounds |  |  |  |  |
| Heavy metals |  | Chromium (VI)- compounds | 18540-29-9 | Directive, 2003/53/EC, 2004-71-D, 2006-90-D Carc.Cat.2, R49; R43; N, R50/53 |
|  |  | Chromium | 7440-47-3 | Directive 80/68/EEC, 1999-263-A, 2004-71-D, 2005-735-FIN,2006-90-D, 2006-223-E, 2006-557-NL, 2007-385-A, 2007-653-A |
|  |  | Copper | 7440-50-8 | Directive 80/68/EEC, 2006-557-NL, 2006-90-D, 2007-385-A, 2007-653-A |
|  |  | Cobalt | 7440-48-4 | Directive 80/68/EEC, 2006-557-NL, 2006-90-D R42/43, R53 |
|  |  | Molybdenum | 7439-98-7 | Directive 80/68/EEC, 2006-557-NL, 2005-735-FIN, 2006223-E, 2006-90-D |
|  |  | Selenium | 7782-49-2 | Directive 80/68/EEC, 2006-557-NL, 2006-90-D, 2005-735-FIN, 2006223-E T, R23/25; R33, R53 |
|  |  | Vanadium | 7440-62-2 | Directive 80/68/EEC, 2006-557-NL, 2006-90-D, 2005-735-FIN, 2004-71-D |
|  |  | Zinc | 7440-66-6 | Directive 80/68/EEC, 2006-557-NL, 2006-90-D, 2005-735-FIN, 2004-71-D, 2006-223-E N, R50/53 |
|  |  | Thallium | 7440-28-0 | Directive 80/68/EEC, 2006-90-D T+, R26/28; R33, R53 |
|  |  | Lead and its compounds | 7439-92-1 | Directive 2000/60/EC, Directive 80/68/EEC, 2004-71-D, 2006223-E, 2006-557-NL, 2006-90-D, 1999-263-A, 2005-735-FIN, Repr. Cat. 1, R61; Repr. Cat. 3, R62; Xn, R20/22; R33; N; R50-53 |
|  |  | Cadmium | 7440-43-9 | Directive 2000/60/EC, Directive 80/68/EEC, 2006-557-NL, 2006-90-D, 2005-735-FIN, 2004-71-D, 2006223-E Carc. Cat. 2, R45; Muta. Cat. 3, R68; Repr. Cat. 3, R62-63; T, R48/23/25; T+; R26; N, R50-53 |
|  |  | Nickel and its compounds | 7440-02-0 | Directive 2000/60/EC, Directive 80/68/EEC, 2006-557-NL, 2006-90-D, 2005-735-FIN, 2004-71-D, 2006223-E Carc. Cat. 3, R40; Xn, R43. |
|  |  | Mercury and its compounds | 7439-97-6 | Directive 2000/60/EC, Directive 80/68/EEC, 2006-557-NL, 2006-90-D, 2005-735-FIN, 2004-71-D, 2006223-E T, R23; R33; N, R50/53 |
|  |  | Tin | 7440-31-5 | Directive 80/68/EEC, 2006-90-D |
| Semimetals |  | Antimony | 7440-36-0 | Directive 80/68/EEC, 2006-90-D, 2005-735-FIN, 2006-223-E Antimony compounds: Xn, R20/22; N, R51-53 |
|  |  | Arsenic | 7440-38-2 | Directive 80/68/EEC, 2006-223-E, 2004-71-D, 2006-557-NL, 2006-90-D, 2005-735-FIN T, R23/25; N, R50/53 |
|  |  | Boron | 7440-42-8 | Directive 80/68/EEC, 2006-90-D |
| Alkaline-earth metals |  | Barium | 7440-39-3 | Directive 80/68/EEC, 2006-90-D, 2006-557-NL, 2005-735-FIN, 2006223-E |
| Biocides |  |  |  |  |
| Chlorinated biocides |  | Pentachlorophenol (PCP) | 87-86-5 | Directive 76/769/EEC, Commission Decision 1994/783/EC, 2006-557-NL Carc.Cat.3, R40; T+, R26; T, R24/25; Xi, R36/37/38; N, R50/53 |
|  |  | Hexachlorobenzene | 118-74-1 | Directive 2000/60/EC, 2006-557-NL, 2006-90-D Carc.Cat.2, R45; T, R48/25; N, R50/53 |
|  |  | Diuron | 330-54-1 | Directive 2000/60/EC Carc.Cat.3, R40; Xn, R22-48/22; N, R50/53 |
|  |  | Pentachlorophenol (PCP) | 87-86-5 | Directive 76/769/EEC, Commission Decision 1994/783/EC, 2006-557-NL Carc.Cat.3, R40; T+, R26; T, R24/25; Xi, R36/37/38; N, R50/53 |
| Other biocides |  |  | Not applicable | Directive 98/8/EC, Commission Decisions 2007/565/EC, 2007/598/EC, 2008/681/EC, 2008/809/EC |

A-2.	Further regulated parameters

Substances and environmental parameters regulated in the EU member states for construction products in addition to those listed as main pollutants in Annex VIII of DIRECTIVE 2000/60/EC.

Table A-2. Further regulated parameters in context of release into soil and/or water.

| Substance group or parameter | Substance or substance subgroup / parameter details | Chemical Abstract Number (CAS) | Regulations / Exemplary Notifications / Classifications |
| --- | --- | --- | --- |
| Inorganic indicator parameters |  |  |  |
| Nitrogen | Ammonium-N | Not applicable Not applicable | Directive 80/68/EEC, Directive 2006/118/EC, 2004-71-D.2007-385-A, 2007-653-A |
|  | Nitrite-N |  | 2007-385-A, 2007-653-A |
| Anions | Chloride |  | Directive 2006/118/EC, 2005-735-FIN, 2004-71-D, 2006-557-NL, 2006-90-D, 2006223-E |
|  | Bromide |  | 2006-557-NL |
|  | Fluoride |  | Directive 80/68/EEC, 2005-735-FIN, 2004-71-D, 2006-557-NL, 2006-90-D, 2006223-E |
|  | Sulphate |  | Directive 2006/118/EC. 2005-735-FIN, 2004-71-D, 2006-557-NL, 2006-90-D, 2006223-E, 2007-385-A, 2007-653-A |
| Chemical sum parameters, characterisation parameters, chemical and biological tests, radioactivity |  |  |  |
| Organic carbon | TOC (total organic carbon) | Not applicable | 2005-735-FIN, 2006-90-D, 2004-71-D TOC tests are a basic part of the characterisation of materials |
|  | DOC (dissolved organic carbon) |  | 2005-735-FIN, 2006-90-D, 2004-71-D DOC tests are a basic part of the characterisation of materials |
| Substances which have a deleterious effect on the taste and/or odour of groundwater, and compounds liable to cause the formation of such substances in such water and to render it unfit for human consumption. |  |  | Directive 80/68/EEC, 2006-90-D. |
| pH |  |  | 1999-263-A, 2007-385-A, 2004-71-D, 2006-90-D |
| Electric conductivity |  |  | 2004-71-D, 2007-385-A, 1999-263-A, 2006-90-D |
| Aerobic biodegradability |  |  | 2006-90-D |
| Anaerobic biodegradability |  |  | 2006-90-D |
| Ecotoxicity |  |  | 2006-90-D |
| Radioactivity |  |  | Radiation Protection Ordinances in different member states (notified according to the EURATOM Treaty) |

A-3.	Other substances deemed relevant: PBT substances

Substances identified before the entering into force of the REACH legislation as persistent, bioaccumulative and toxic (PBT) or very persistent and very bioaccumulative (vPvB) according to the criteria of the Technical Guidance Document in support of Commission Regulation (EC) No 1488/94 on Risk Assessment for existing substances (for further information see: http://ecb.jrc.ec.europa.eu/esis/index.php?PGM=pbt).

Further evaluation will take place under the framework of REACH. The substances marked bold are included on the Candidate List of Substances of Very High Concern for authorisation. All PBT and vPvB substances are potential future SVHC. For the so far existing regulations see table A-1. The substances are listed here for information, although not all of them may be relevant for construction products.

Table A-3. Substances identified as PBT or vPvB according to EU criteria.

|  | CAS number | Substance name | Criteria fulfilment |
| --- | --- | --- | --- |
|  | 87-61-6 | 1,2,3 trichlorobenzene | fulfilling PBT criteria |
|  | 120-82-1 | 1,2,4 trichlorobenzene | fulfilling PBT criteria |
|  | 85535-84-8 | Alkanes, C10-13, chloro | fulfilling PBT criteria |
|  | 90640-80-5 | Anthracene oil | fulfilling PBT criteria |
|  | 90640-81-6 | Anthracene oil, anthracene paste | fulfilling PBT criteria |
|  | 91995-15-2 | Anthracene oil, anthracene paste, anthracene fraction | fulfilling PBT criteria |
|  | 91995-17-4 | Anthracene oil, anthracene paste, distn. Lights | fulfilling PBT criteria |
|  | 90640-82-7 | Anthracene oil, anthracenelow | fulfilling PBT criteria |
|  | 120-12-7 | Anthracene, pure | fulfilling PBT criteria |
|  | 56-35-9 | Bis(tributyltin)oxide (TBTO) | fulfilling PBT criteria |
|  | 294-62-2 | Cyclododecane | fulfilling PBT criteria |
|  | 32536-52-0 | Diphenyl ether, octabromo derivative | fulfilling PBT criteria |
|  | 90640-86-1 | Distillates (coal tar), heavy oils | fulfilling PBT criteria |
|  | 91995-42-5 | Distillates (coal tar), heavy oils, pyrene fraction | fulfilling PBT criteria |
|  | 91995-52-7 | Distillates (coal tar), pitch, pyrene fraction | fulfilling PBT criteria |
|  | 115-29-7 | Endosulfan | fulfilling PBT criteria & POP |
|  | 25637-99-4 | Hexabromocyclododecane | fulfilling PBT criteria |
|  | 87-68-3 | Hexachlorobuta1,3-diene | fulfilling PBT & vPvB criteria & POP |
|  | 1836-75-5 | Nitrofen | fulfilling PBT criteria |
|  | 133-49-3 | Pentachlorobenzenethiol | fulfilling PBT & vPvB criteria |
|  | 65996-93-2 | Pitch, coal tar, hightemp. | fulfilling PBT criteria |
|  | 92061-94-4 | Residues (coal tar), pitch distn. | fulfilling PBT criteria |
|  | 75-74-1 | Tetramethyllead | fulfilling PBT criteria |
|  | 81-15-2 | 5-tert-butyl-2,4,6-trinitro-m-xylene (musk xylene) | identified as a vPvB (evaluation under REACH) |

Annex B

List B “Indoor Air”

Explanation of used colours:

blue = regulation for content

yellow = regulation for release

purple = regulation for content and release

grey = regulation for indoor air quality

green = PBT-substance (persistent, bioaccumulative and toxic; see also table A-3)

red = CMR-substances (carcinogenic mutagenic or toxic for reproduction) of EU categories 1 and 2

List B-1.	Regulated dangerous substances and further associated parameters

List B-1 gives an overview of the regulated substances and parameters relevant for construction products used in interiors. A detailed list of the regulated volatile organic compounds is given in table B-2 and a detailed list of the regulated volatile carcinogens in list B-3.

Table B-1. Regulated dangerous substances and further parameters in context of emissions into indoor air

| Substance group or parameter | Substance or substance subgroup | Chemical Abstract Number (CAS) | Regulations / Exemplary Notifications / Classifications |
| --- | --- | --- | --- |
| Very volatile, volatile and semi volatile organic compounds (VVOC/ VOC /SVOC) |  |  |  |
| VVOC | Formaldehyde | 50-00-0 | T; R23/24/25 / C; R34 / Carc.Cat.3; R40 /R43 Note: IARC (International Agency for the Research of Cancer, part of under WHO) has classified formaldehyde as carcinogenic to humans 2004. 1985-26-D, 1987-125-F, 1996-PL, 1997/346/NL, 1998/57/NL, 2005-255-D, 2007-90-DK, 2007-372-FIN, 2008-243-S, 2008-321-GR |
|  | Substances according to the definitions in footnote 9, e.g. aliphatic hydrocarbons, aliphatic alcohols, amines, esters, acetone and aldehydes | See annex B-2 | 2007-372-FIN, 2005-255-D 2004/42/EC Directive 76/769/EEC (carcinogenic substances, clause 29) Priority VVOCs are substances classified as carcinogenic (see list in Annex B-3), mutagenic or toxic for reproduction (so called CMR substances of categories 1 and 2 according to Regulation (EC) Nr. 1272/2008). |
| VOC9 | Benzene | 71-43-2 | DIR 76/769/EEC, 2007-372-FIN, 1995-213-A, 2009-34-F F; R11 / Carc.Cat.1; R45 / Muta.Cat.2; R46 / T; R48/23/24/25 / Xn; R65 / Xi; R36/38 |
|  | Styrene | 100-42-5 | 2007-372-FIN, 1996-PL R10 / Xn; R20 / Xi; R36/38 |
|  | Phenolic compounds | Not applicable | 76/769/EEC |
|  | Substances according to the definitions in footnote 9, e.g. aromatic hydrocarbons, aliphatic hydrocarbons, terpenes, amines, aliphatic alcohols and ether, aromatic alcohols, glycols, glycolethers, aldehydes, ketones, acids, esters and lactones, chlorinated hydrocarbons, siloxanes, isothiazolones (CIT/MIT/BIT), phenols, cresols, naphthalene. | See annex B-2 | 2005-255-D, 2001-200-A, 2004/42/EC, 1995-213-A 2002-271-FIN Both, emissions of individual VOC and the sum emission of all substances are considered. Priority VOCs are substances classified as carcinogenic (see annex B-3), mutagenic or toxic for reproduction (so called CMR substances of categories 1 and 2 according to Regulation (EC) Nr. 1272/2008). Note: A current list of regulated VOC is given in part B-2 of this document. The concentration of not identified and non-assessable VOCs is considered separately. |
| SVOC9 | Dibutyl pthalate | 84-74-2 | 2009-104-F, Regulation 1907/2006 (Candidate List: SVHC) Repr.Cat.2; R61 / Repr.Cat.3; R62 / N; R50 |
|  | Substances according to the definitions in footnote 9, e.g. phthalates, aliphatic hydrocarbons, organophosphorous compounds. See also note 2. See annex B-2 and B-3 | 2005-255-D | Both, emissions of individual SVOCs and the sum emission of all SVOC need to be considered. Priority SVOCs are substances classified as carcinogenic (see annex B-3), mutagenic or toxic for reproduction (so called CMR substances of categories 1 and 2 according to Regulation (EC) Nr. 1272/2008). |
| Biocides, flame retardants and other organic substances for which (also) content-based restrictions exist |  |  |  |
| Pentachlorophenol (PCP) |  | 87-86-5 | Directive 76/769/EEC, Commission Decisions 1994/783/EC, 1999/831/EC, 1996/211/EC, 2001-450-D, 2007-372-FIN, 1996-PL Carc.Cat.3; R40 / T+; R26 / T; R24/25 / Xi; R36/37/38 / N; R50/53 |
| DDT (p,p ′-Dichlor-2,2-diphenyl-1,1,1-trichlorethan) |  | 50-29-3 | Directive 79/117/EEC, 2007-372-FIN T; R25-48/25 / Carc.Cat.3; R40 / N; R50/53 |
| Wood preservatives |  | Not applicable | 1996-183-D The use of some wood-preservatives is not allowed for special intended uses, e.g. in living areas. |
| Polycyclic aromatic hydrocarbons – 16 PAH |  | 83-32-9 208-96-8 120-12-7 56-55-3 50-32-8 205-99-2 191-24-2 207-08-9 218-01-9 53-70-3 206-44-0 86-73-7 193-39-5 91-20-3 85-01-8 129-00-0 | Directive 76/769/EEC (tar oils). The individual substances which are considered differ in the different member states. From the analytical point of view tests are usually based on the list of US-EPA. This list includes 16 PAH. Anthracene is on the Candidate List of Substances of Very High Concern under REACH. 2002-271-FIN (Benzo(a)pyrene, naphthalene) 1996-PL (naphthalene) |
| Polychlorinated biphenyls (PCB) |  | 1336-36-3 | Directive 76/769/EEC, 2001-450-D, 2007-372-FIN Xn; R33 / N; R50/53 |
| Polybrominated diphenylethers | Penta-bromodiphenylether | 32534-81-9 | Directive 76/769/EEC Xn; R48/21/22 / R64 / N; R50/53 |
|  | Octadibromodiphenylether | 32536-52-0 | Directive 76/769/EEC Repr.Cat.2; R61 / Repr.Cat.3; R62 |
|  | Decabromodiphenylether | 1163-19-5 | 2005-255-D, 2005-9020-N R53 |
| Azocolourants: A mixture of: disodium (6-(4-anisidino)-3-sulfonato-2-(3,5-dinitro-2-oxidophenylazo)-1-naphtholato)(1-(5-chloro-2-oxidophenylazo)-2-naphtholato)chromate(1-);trisodium bis(6-(4-anisidino)-3-sulfonato-2-(3,5-dinitro-2-oxidophenylazo)-1-naphtholato)chromate(1-) |  | Not available | Directive 76/769/EEC R43 / N; R50-53 |
| Di(2-ethyhexyl)phthalate (DEHP) |  | 117-81-7 | 2009-104-F, 2007-372-FIN, 2007-9016-N, Regulation 1907/2006 (Candidate List: SVHC) Repr.Cat.2; R60-61 |
| Benzyl butyl phthalate |  | 85-68-7 | Regulation 1907/2006 (Candidate List: SVHC) Repr.Cat.2; R61 / Repr.Cat.3; R62 / N; R50/53 |
| Polyhalogenated dibenzo-p-dioxins and polyhalogenated dibenzofuranes |  | Not applicable | 1993/141/D 25 individual substances are prohibited above certain concentrations. The individual substances are classified differently. |
| Perfluorooctanesulfonate (PFOS) |  | 45298-90-6 | Directive 76/769/EEC The individual substances are classified differently. |
| Partially fluorinated and fully fluorinated hydrocarbons, perfluorocarbons (HFCs, FCs, PFCs) |  | Not applicable | 2001-121-DK, 2002-37-A |
| Inorganic substances |  |  |  |
| Ammonia |  | 7664-41-7 | 2005-255-D, 2002-271-FIN, 1996-PL R10 / T; R23 / C; R34 / N; R50 |
| Hydrazine |  | 302-01-2 | 2005-255-D, -2002-271-FIN R10 / Carc.Cat.2; R45 / T; R23/24/25 / C; R34 / R43 / N; R50/53 |
| Halogens | Chloride Fluoride | Not applicable | 2001-450-D The content of chloride and fluoride is restricted in recycled wood used for the production of wood-based panels. |
| Metals for which content restrictions apply to construction products used in interiors |  |  |  |
|  | Cadmium and its compounds | 7440-43-9 | Directive 76/769/EEC, 2001-450-D, 2001-200-A, 2007-372-FIN Carc. Cat. 2, R45; Muta. Cat. 3, R68; Repr. Cat. 3, R62-63; T, R48/23/25; T+; R26; N, R50-53 |
|  | Arsenic | 7440-38-2 | Directive 76/769/EEC, 2001-450-D, 2007-372-FIN T; R23/25 / N; R50/53 |
|  | Copper | 7440-50-8 | 2001-450-D |
|  | Lead | 7439-92-1 | 2001-450-D, 2001-200-A T; R 61 / R 20/22 / R 33 / N; R 50/53 / R 62 |
|  | Mercury | 7439-97-6 | 2001-450-D, 2001-200-A, 2007-372-FIN, 1996-PL T; R23 / R33 / N; R50/53 |
|  | Chromium | 7440-47-3 | 2001-450-D, 2002-9039-N, 2007-372-FIN |
|  | Chromium (VI) | 18540-29-9 | 2001-200-A Carc.Cat.2; R49 / R43 / N; R50/53 |
| Particles / Fibres |  |  |  |
| Asbestos | Actinolite | 77536-66-4 | Directive 76/769/EEC, 1997-527-DK, 1995188-DK, 2004-294-NL, 2004-410-NL, 1996-PL 2007-372-FIN Carc.Cat.1; R45 / T; R48/23 |
|  | Amosite | 12172-73-5 |  |
|  | Anthophyllite | 77536-67-5 |  |
|  | Chrysotile | 12001-29-5 |  |
|  | Crocidolite | 12001-28-4 |  |
|  | Tremolite | 77536-68-6 |  |
| Synthetic vitreous (silicate) fibres |  | Not applicable | Directive 76/769/EEC. Refractory ceramic fibres [Man-made vitreous (silicate) fibres with random orientation with alkaline oxide and alkali earth oxide (Na2O + K2O + CaO + MgO + BaO) content less or equal to 18 % by weight] are classified as carcinogenic (Cat. 2). 1998-156-D, 1997-527-DK, 1995188-DK Mineral Wool Man-made vitreous (silicate) fibres with random orientation with alkaline oxide and alkali earth oxide (Na2O+K2O+CaO+ MgO+BaO) content greater than 18 % by weight] Carcinogenic Cat. 3 with the exception of those specified elsewhere in the Annex of directive 97/69 EC (Nota Q). In Germany the exoneration criteria differ in some aspect from Nota Q. Mineral Wool not fulfilling these criteria is banned |
| Radiation |  |  |  |
| Radioactivity | Naturally occurring radionuclides | Not applicable | 1997-527-DK, 1995188-DK, 2002-271-FIN, German Radiation Protection Ordinance (notified under the EURATOM Treaty). European Commission Radiation Protection Guide 112 In Germany the activity concentration of U-238 and Th-232 is used for the definition of residues which have to be supervised, A design level is recommended for the radon gas concentration in dwellings. |

B-2.	Regulated volatile organic compounds (VVOC, VOC, SVOC) excluding carcinogenic substances of category 1 and 2 (see table B-3).

The substances in the table are derived from the following notifications:

Column G: 2009-167-D / substances for which lowest concentrations of interest (LCI values) have been defined

Column F: 2007-372-FIN / substances for which a maximum content of impurities in normal premises is derived from occupational exposure limits (substances explicitly not used in construction products have been excluded)

Column P: Polish ordinance on the permitted concentrations and intensities of agents harmful for health emitted by construction materials in rooms intended for human residence (1996-PL)

Explanation of used colours:

Red = MR substance (mutagenic / toxic for reproduction) of category 1 or 2 (potential future SVHC under REACH)

Pink = very volatile organic compound (VVOC; boiling point below 68 °C. Highlighted due to technical issues related to the choice of the suitable test method)

Table B-2. VOC regulated as emissions from construction products into or concentrations in indoor air in buildings

| Substance | CAS No. | Classifications | G | F | P |
| --- | --- | --- | --- | --- | --- |
| Aromatic hydrocarbons |  |  |  |  |  |
| Toluene | 108-88-3 | F; R11 / Repr.Cat.3; R63 / Xn; R48/20-65 / Xi; R38 / R67 | x | x | x |
| m-Nitrotoluene | 99-08-1 | T; R 23/24/25 |  | X |  |
| 4-Nitrotoluene | 99-99-0 | T; R23/24/25 / R33 / N; R51/53 |  | X |  |
| 4-tert-Butyltoluene | 98-51-1 | T; R23 / R22 / R36 / N; R51/53 |  | X |  |
| Ethyl benzene | 100-41-4 | F; R11 / Xn; R20 | x | x | X |
| Xylene, mix of o-, m- and p-xylene isomers | 1330-20-7 | R10 / Xn; R20/21 / Xi; R38 | X | x | x |
| p-Xylene | 106-42-3 | R10 / Xn; R20/21 / Xi; R38 | x | x | X |
| m-Xylene | 108-38-3 | R10 / Xn; R20/21 / Xi; R38 | X | x | x |
| o-Xylene | 95-47-6 | R10 / Xn; R20/21 / Xi; R38 | x | x | X |
| Cumene | 98-82-8 | R10 / Xn; R65 / Xi; R37 / N; R51/53 | x | X |  |
| n-Propyl benzene | 103-65-1 | R10 / Xn; R65 / Xi; R37 / N; R51/53 | X |  |  |
| 1-Propenyl benzene (ß-methyl styrene) | 637-50-3 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| 1.3.5-Trimethylbenzene | 108-67-8 | R10 / Xi; R37 / N; R51/53 | x | X |  |
| 1.2.4-Trimethylbenzene | 95-63-6 | R10 / Xn; R20 / Xi; R36/37/38 / N; R51/53 | X | x |  |
| 1.2.3-Trimethylbenzene | 526-73-8 | R10 / Xi; R37 | x | X |  |
| 2-Ethyltoluene | 611-14-3 | R10 / Xn; R65 | X |  |  |
| 1-Isopropyl-2-methylbenzene (o-cymene) | 527-84-4 | R10 / Xn; R20 | X |  |  |
| 1-Isopropyl-3-methylbenzene (m-cymene) | 535-77-3 | R10 | X |  |  |
| 1-Isopropyl-4-methylbenzene (p-cymene) | 99-87-6 | R10 / Xi; R36/37/38 | X |  |  |
| 1.2.4.5-Tetramethyl benzene | 95-93-2 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| n-Butyl benzene | 104-51-8 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| 1.3-Diisopropylbenzene | 99-62-7 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| 1.4-Diisopropylbenzene | 100-18-5 | Xi; R36/37/38 | X |  |  |
| Divinylbenzene | 1321-74-0 | Xi; R36/37/38 / N; R51/53 |  | X |  |
| Nitrobenzene | 98-95-3 | Carc.Cat.3; R40 / Repr.Cat.3; R62 / T; R23/24/25-48/23/24 / N; R51/53 |  | X |  |
| Benzene-1,2,4-tricarboxylic acid 1,2-anhydride | 552-30-7 | Xi; R37-41 / R42/43 |  | X |  |
| Phenyl octane and isomers | 2189-60-8 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified.. | X |  |  |
| 1-Phenyldecane and isomers | 104-72-3 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified.. | X |  |  |
| 1-Phenyl undecane and isomers | 6742-54-7 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified.. | X |  |  |
| 4-Phenyl cyclohexene (4-PCH) | 4994-16-5 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified.. | X |  |  |
| Styrene | 100-42-5 | R10 / Xn; R20 / Xi; R36/38 | x | X |  |
| Phenyl acetylene | 536-74-3 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified.. | X |  |  |
| 2-Phenylpropene (α-Methylstyrene) | 98-83-9 | R10 / Xi; R36/37 / N; R51/53 | x | X |  |
| Vinyl toluene (all isomers: o-,m-,p-methyl styrenes) | 25013-15-4 | The individual isomers are classified differently | X | x |  |
| 2-Methylstyrene | 611-15-4 | Xn; R20 / N; R51/53 |  | X |  |
| 3-Methylstyrene | 100-80-1 | R10 / Xn; R20/65 / R 36/37/38 |  | X |  |
| 4-Methylstyrene | 622-97-9 | R10 / Xn; R65 / R 36/37/38 |  | X |  |
| Other alkylbenzenes, as long as individual. isomers have not to be evaluated differently |  |  | x |  |  |
| Naphthalene | 91-20-3 | Carc.Cat.3; R40 / Xn; R22 / N; R50/53 | x | X |  |
| Indene | 95-13-6 | R10 / Xn; R65 | X | x |  |
| Chloronaphtalene | 90-13-1 | R 22 / R 36/37/38 |  |  | X |
| Aniline | 62-53-3 | Carc.Cat.3; R40 / Muta.Cat.3; R68 / T; R23/24/25-48/23/24/25 / Xi; R41 / R43 / N; R50 |  | X |  |
| p-Anisidine | 104-94-9 | T+; R26/27/28 / R33 / N; R50 |  | X |  |
| Biphenyl | 92-52-4 | Xi; R36/37/38 / N; R50/53 |  | X |  |
| N,N'-Dimethylaniline | 121-69-7 | Carc.Cat.3; R40 / T; R23/24/25 / N; R51/53 |  | X |  |
| N,N-Dimethylacetamide | 127-19-5 | Repr.Cat.2; R61 / Xn; R20/21 |  | X |  |
| Phenylphosphine | 638-21-1 | F; R17 / Xn; R20 / R 36/37/38 |  | X |  |
| p-Phenylenediamine | 106-50-3 | T; R23/24/25 / Xi; R36 / R43 / N; R50/53 |  | X |  |
| Phenyl isocyanate | 103-71-9 | R 10 / R 22 / T+; R 26 / R 34 / R 37 / R 42/43 |  | X |  |
| Phenylmercaptan | 108-98-5 | R 10 / R 24/25 / T+; R 26 / R 36/38 |  | X |  |
| Phthalic anhydride | 85-44-9 | Xn; R22 / Xi; R37/38-41 / R42/43 |  | x | X |
| Hexahydro-1,3-isobenzofurandione | 85-42-7 | Xi; R41 / R42/43 |  | X |  |
| Hexahydromethylphthalic acid anhydride | 25550-51-0 | Xi; R41 / R42/43 |  | X |  |
| 1,4-Dihydroxybenzene | 123-31-9 | Carc.Cat.3; R40 / Muta.Cat.3; R68 / Xn; R22 / Xi; R41 / R43 / N; R50 |  | X |  |
| Chlorobenzene | 108-90-7 | R10 / Xn; R20 / N; R51/53 |  | x | X |
| Chlorotoluene, mixed isomeres | 25168-05-2 | Xn; R20 / N; R51/53 |  | X |  |
| Cresol mixed isomers | 1319-77-3 | T; R24/25 / C; R34 |  | x | X |
| m-Phenylenebis(methylamine) | 1477-55-0 | R 20/21/22 / C; R34 |  | X |  |
| 2,3-Dimethylaniline | 87-59-2 | T; R23/24/25 / R33 / N; R51/53 |  | X |  |
| 2,4-Dimethylaniline | 95-68-1 | T; R23/24/25 / R33 / N; R51/53 |  | X |  |
| 2,5-Dimethylaniline | 95-78-3 | T; R23/24/25 / R33 / N; R51/53 |  | X |  |
| 2,6-Dimethylaniline | 87-62-7 | Carc.Cat.3; R40 / Xn; R20/21/22 / Xi; R37/38 / N; R51/53 |  | X |  |
| 3,4-Dimethylaniline | 95-64-7 | T; R23/24/25 / R33 / N; R51/53 |  | X |  |
| 3,5-Dimethylaniline | 108-69-0 | T; R23/24/25 / R33 / N; R51/53 |  | X |  |
| 4-Nitroaniline | 100-01-6 | T; R23/24/25 / R33 / R52/53 |  | X |  |
| Parathion | 56-38-2 | T+; R26/28/ T; R24-48/25 / N; R50/53 |  | X |  |
| Pyrocatechol | 120-80-9 | Xn; R21/22 / Xi; R36/38 |  | X |  |
| Resorcinol | 108-46-3 | Xn; R22 / Xi; R36/38 / N; R50 |  | X |  |
| Aliphatic hydrocarbons |  |  |  |  |  |
| Nitromethane | 75-52-5 | R5 -10 / Xn; R22 |  | X |  |
| Nitroethane | 79-24-3 | R10 / Xn; R20/22 |  | X |  |
| 1-Nitropropan | 108-03-2 | R10 / Xn; R20/21/22 |  | X |  |
| Propane | 74-98-6 | F+; R12 |  | X |  |
| 2-Methylbutane | 78-78-4 | F+; R12 / Xn; R65 / R66 / R67 /N; R51/53 |  | X |  |
| 2,3-Dimethylbutane | 79-29-8 | F; R11 / Xn; R65 / Xi; R38 / R67 / N; R51/53 |  | X |  |
| n-Pentane | 109-66-0 | F+; R12 / Xn; R65 / R66 / R67 / N; R51/53 |  | X |  |
| 2-Methylpentane | 107-83-5 | F; R11 / Xn; R65 / Xi; R38 / R67 / N; R51/53 |  | X |  |
| 3-Methylpentane | 96-14-0 | F; R11 / Xn; R65 / Xi; R38 / R67 / N; R51/53 |  | X |  |
| n-Hexane | 110-54-3 | F; R11 / Repr.Cat.3; R62 / Xn; R48/20-65 / Xi; R38 / R67 / N; R51/53 | x | X |  |
| Cyclohexane | 110-82-7 | F; R11 / Xn; R65 / Xi; R38 / R67 / N; R50/53 | x | x | X |
| Methyl cyclohexane | 108-87-2 | F; R11 / Xn; R65 / Xi; R38 / R67 / N; R51/53 | x | X |  |
| Cyclohexene | 110-83-8 | F; R 11 / Xn; R 21/22 |  | X |  |
| n-Heptane | 142-82-5 | F; R11 / Xn; R65 / Xi; R38 / R67 / N; R50/53 | x | X |  |
| 2,2-Dimethylpentane | 590-35-2 | F; R11 / Xn; R65 / Xi; R38 / R67 / N; R50/53 |  | X |  |
| 2,3-Dimethylpentane | 565-59-3 | F; R11 / Xn; R65 / Xi; R38 / R67 / N; R50/53 |  | X |  |
| 2,4-Dimethylpentane | 108-08-7 | F; R11 / Xn; R65 / Xi; R38 / R67 / N; R50/53 |  | X |  |
| 3,3-Dimethylpentane | 562-49-2 | F; R11 / Xn; R65 / Xi; R38 / R67 / N; R50/53 |  | X |  |
| 3-Ethylpentane | 617-78-7 | F; R11 / Xn; R65 / Xi; R38 / R67 / N; R50/53 |  | X |  |
| 2-Methylhexane | 591-76-4 | F; R11 / Xn; R65 / Xi; R38 / R67 / N; R50/53 |  | X |  |
| 3-Methylhexane | 589-34-4 | F; R11 / Xn; R65 / Xi; R38 / R67 / N; R50/53 |  | X |  |
| 2,2,3-Trimethylbutane | 464-06-2 | F; R11 / Xn; R65 / Xi; R38 / R67 / N; R50/53 |  | X |  |
| other saturated n-aliphatic hydrocarbons until C8 |  |  | X |  |  |
| other saturated n-aliphatic hydrocarbons from C9 |  |  | X |  |  |
| Methylamine anhydrous | 74-89-5 | F+; R12/ Xn; R20 / Xi; R37/38-41 |  | X |  |
| Trimethylamine | 75-50-3 | F+; R12 / Xn; R20 / Xi; R37/38-41 |  | X |  |
| Di-n-butylamine | 111-92-2 | R10 / Xn; R20/21/22 |  | X |  |
| Ethylamine | 75-04-7 | F+; R12 / Xi; R36/37 |  | X |  |
| Diethylenetriamine | 111-40-0 | Xn; R21/22 / C; R34 / R43 |  | X |  |
| Diethylamine | 109-89-7 | F; R11 / Xn; R20/21/22 / C; R35 |  | X |  |
| Ethylenediamine | 107-15-3 | R10 / C; R34 / Xn; R21/22 / R42/43 |  | X |  |
| Diisopropylamine | 108-18-9 | F; R11 / Xn; R20/22 / C; R34 |  | X |  |
| Diisopropyl ether | 108-20-3 | F; R11 / R19 / R66 / R67 |  | X |  |
| Propylamine | 107-10-8 | F; R 11 / R 20/21/22 / C; R 34 |  | X |  |
| Isopropylamine | 75-31-0 | F+; R12 / Xi; R36/37/38 |  | X |  |
| Cyclohexylamine | 108-91-8 | R10 / Repr.Cat.3; R62 / Xn; R21/22 / C; R34 |  | X |  |
| 5-Ethylidene-2-norbornene | 16219-75-3 | R10 / Xi; R36/37/38 |  | X |  |
| Tert-Butyl ethyl ether | 637-92-3 | F; R 11 / Xi; R 36/38 |  | X |  |
| Dipropyl ether | 111-43-3 | F; R11 / R19 / R66 / R67 |  | X |  |
| Methyl isocyanate | 624-83-9 | F+; R12 / Repr.Cat.3; R63 / T+; R26 / T; R24/25 / R42/43 / Xi; R37/38-41 |  | X |  |
| n-Octane | 111-65-9 | F; R11 / Xn; R65 / Xi; R38 / R67 / N; R50/53 |  | X |  |
| 1,1,2,2-Tetrabromoethane | 79-27-6 | T+; R26 / Xi; R36 / R52/53 |  | X |  |
| Terpenes |  |  |  |  |  |
| 3-Carene | 498-15-7 | R10 / Xi; R43 / N; R50/53 | X |  |  |
| -Pinene | 80-56-8 | R 10 / Xn; R 20/21/22 / R 36/38 / R 43 / N; R 51/53 / R 65 | X |  |  |
| ß-Pinene | 127-91-3 | R 10 / Xn; R 20/21/22 / R 36/38 / R 43 / N; R 51/53 / R 65 | X |  |  |
| Limonene | 138-86-3 | R10 / Xi; R38 / R43 / N; R50/53 | x | X |  |
| Dipentene | 5989-27-5 | R10 / Xi; R38 / R43 / N; R50/53 |  | X |  |
| Other terpene hydrocarbons |  |  | x |  |  |
| Turpentine, oil | 8006-64-2 | R10 / Xn; R20/21/22-65 / Xi; R36/38 / R43 / N; R51/53 |  | x |  |
| Aliphatic alcohols |  |  |  |  |  |
| Methanol | 67-56-1 | F; R11 / T; R23/24/25-39/23/24/25 |  | X |  |
| Ethanol | 64-17-5 | F; R11 |  | X |  |
| Propan-1-ol | 71-23-8 | F; R11 / Xi; R41 / R67 |  | X |  |
| Propan-2-ol | 67-63-0 | F; R11 / Xi; R36 / R67 |  | X |  |
| 2-Propen-1-ol | 107-18-6 | R10 / T; R23/24/25 / Xi; R36/37/38 / N; R50 |  | X |  |
| Prop-2-yn-1-ol | 107-19-7 | R10 / T; R23/24/25 / C; R34 / N; R51-53 |  | X |  |
| 2-methylpropan-2-ol | 75-65-0 | F; R11 / Xn; R20 | x | X |  |
| 2-Methyl-1-propanol | 78-83-1 | R10 / Xi; R37/38-41 / R67 | x | X |  |
| Butan-1-ol | 71-36-3 | R10 / Xn; R22 / Xi; R37/38-41 / R67 | x | X | X |
| 2-Butanol | 78-92-2 | R10 / Xi; R36/37 / R67 |  | X |  |
| (S)-Butan-2-ol | 4221-99-2 | R10 / Xi; R36/37 / R67 |  | X |  |
| (R)-Butan-2-ol | 14898-79-4 | R10 / Xi; R36/37 / R67 |  | X |  |
| (+-)-Butan-2-ol | 15892-23-6 | R10 / Xi; R36/37 / R67 |  | X |  |
| Pentanol (all isomers) | 71-41-0 30899-19-5 94624-12-1 6032-29-7 584-02-1 137-32-6 123-51-3 598-75-4 75-85-4 75-84-3 | R10 / Xn; R20 / Xi; R37 / R66 R10 / Xn; R20 / Xi; R37 / R66 not classified in the Annex I of Directive 67/548/EEC R10 / Xn; R20 / Xi; R37 / R66 R10 / Xn; R20 / Xi; R37/38 R10 / Xn; R20 / Xi; R37 / R66 R10 / Xn; R20 / Xi; R37 / R66 R10 / Xn; R20 / Xi; R37 / R66 F; R11 / Xn; R20 / Xi; R37/38 F; R11 | x | X |  |
|  |  |  | x |  |  |
|  |  |  | X |  |  |
|  |  |  | X |  |  |
|  |  |  | X |  |  |
|  |  |  | x | X |  |
|  |  |  | x | X |  |
|  |  |  | X |  |  |
|  |  |  | x |  |  |
|  |  |  | X |  |  |
| 1-Hexanol | 111-27-3 | Xn; R22 |  |  |  |
| Cyclohexanol | 108-93-0 | Xn; R20/22 / Xi; R37/38 | x | X |  |
| 2-Ethyl-1-hexanol | 104-76-7 | Xi; R36/38 | X |  |  |
| 1-Octanol | 111-87-5 | Xi; R36 | X |  |  |
| 4-Hydroxy-4-methyl-pentane-2-on (diacetone alcohol) | 123-42-2 | Xi; R36 | x | X |  |
| C4 - C10 saturated n-aliphatic alcohols |  |  | x |  |  |
| Furfuryl alcohol | 98-00-0 | Xn; R20/21/22 |  | X |  |
| 2,3-Epoxypropyl isopropyl ether | 4016-14-2 | Muta.Cat.3; R68 / R10 / R19 / Xn; R20 / R42/43 |  | X |  |
| 2-Methylpentane-2,4-diol | 107-41-5 | Xi; R36/38 |  | X |  |
| 4-Methylpentan-2-ol | 108-11-2 | R10 / Xi; R37 |  | X |  |
| Methylcyclohexanol | 25639-42-3 | R 20 |  | X |  |
| Aromatic alcohols |  |  |  |  |  |
| Phenol | 108-95-2 | Muta.Cat.3; R68 / T; R23/24/25 / Xn; R48/20/21/22 / C; R34 | x | x | X |
| 4-(α,α-dimethylbenzyl)phenol (p-Cumyl phenol) | 599-64-4 | Not listed in the Part 3 of Annex VI to Regulation (EC) No 1272/2008 / Annex 1 of the Directive 67/548/EEC: substances not listed must be self classified. |  |  | X |
| Butylated hydroxytoluene | 128-37-0 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | x | X |  |
| Benzyl alcohol | 100-51-6 | Xn; R20/22 | X |  |  |
| Glycols, Glycolethers |  |  |  |  |  |
| Propylene glycol (1,2-Dihydroxypropane) | 57-55-6 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified.. | X |  |  |
| Ethandiol (Ethylene glycol) | 107-21-1 | Xn; R22 | x | x | X |
| Ethylene glycol-monobutylether | 111-76-2 | Xn; R20/21/22 / Xi; R36/38 | x | X |  |
| Diethylene glycol | 111-46-6 | Xn; R22 | X |  |  |
| Diethylene glycol-monobutylether | 112-34-5 | Xi; R36 | x | X |  |
| 2-Phenoxyethanol | 122-99-6 | Xn; R22 / Xi; R36 | x | X |  |
| Ethylene carbonate | 96-49-1 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified.. | X |  |  |
| 1-Methoxy propanol-2 | 107-98-2 | R10 / R67 | x | X |  |
| 2.2.4-Trimethyl-1.3-pentane diol, monoisobutyrate (Texanol®) | 25265-77-4 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified.. | X |  |  |
| Butyl glycolate | 7397-62-8 | Xi; R41 | X |  |  |
| Diethylene glycol monomethyl ether acetate | 124-17-4 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified.. | X |  |  |
| Dipropylene glycol monomethyl ether | 34590-94-8 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified.. | x | X |  |
| 2-Methoxyethanol | 109-86-4 | R10 / Repr.Cat.2; R60-61 / Xn; R20/21/22 | x | X |  |
| 2-(2-Methoxyethoxy)ethanol | 111-77-3 | Repr.Cat.3; R63 |  | X |  |
| 2-Ethoxyethanol | 110-80-5 | R10 / Repr.Cat.2; R60-61 / Xn; R20/21/22 | x | X |  |
| 2-Propoxyethanol | 2807-30-9 | Xn; R21 / Xi; R36 | X |  |  |
| 2-Methylethoxyethanol | 109-59-1 | Xn; R20/21 / Xi; R36 | X |  |  |
| 2-Hexoxyethanol | 112-25-4 | Xn; R21/22 / C; R34 | X |  |  |
| 1,2-Dimethoxyethan | 110-71-4 | Repr.Cat.2; R60 / Repr.Cat.2; R61 / F; R11 / R19 / Xn; R20 | X |  |  |
| 1,2-Diethoxyethan | 73506-93-1 / 629-14-1 | F; R11 / R19 / Repr. Cat. 2; R61 / Repr. Cat. 3; R62 / Xi; R36 |  |  |  |
| 2-Methoxyethyl acetate | 110-49-6 | Repr.Cat.2; R60-61 / Xn; R20/21/22 | x | X |  |
| 2-Ethoxyethyl acetate | 111-15-9 | Repr.Cat.2; R60-61 / Xn; R20/21/22 | x | X |  |
| 2-Butoxyethyl acetate | 112-07-2 | Xn; R20/21 | x | X |  |
| 2-(2-Hexoxyethoxy)-ethanol | 112-59-4 | Xn; R21 / Xi; R41 | X |  |  |
| 1-Methoxy-2-(2-methoxy-ethoxy)-ethan | 111-96-6 | R10 / R19 / Repr.Cat.2; R60-61 | X |  |  |
| 2-Methoxy-1-propanol | 1589-47-5 | R10 / Repr.Cat.2; R61 / Xi; R37/38-41 | X |  |  |
| 2-Methoxy-1-propyl acetate | 70657-70-4 | R10 / Repr.Cat.2; R61 / Xi; R37 | X |  |  |
| Propilene glycol diacetate | 623-84-7 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified.. | X |  |  |
| Dipropylene glycol | 110-98-5 / 25265-71-8 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified.. | X |  |  |
| Dipropylene glycol-monomethyl ether acetate | 88917-22-0 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified.. | X |  |  |
| Dipropylene glycol-mono-n-propylether | 29911-27-1 | Xi / R36 | X |  |  |
| Dipropylene glycol-ono-n-butylether | 29911-28-2 / 35884-42-5 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified.. | X |  |  |
| Dipropylene glycol-mono-t-butylether | 132739-31-2 (Mixture) | Xi; R 36/37/38 | X |  |  |
| 1,4-Butandiol | 110-63-4 | Xn; R22 | X |  |  |
| Tripropylene glycol-monomethyl ether | 20324-33-8 / 25498-49-1 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | x |  |  |
| Triethylene glycol-dimethyl ether | 112-49-2 | R19 / Repr.Cat.2; R61 / Repr.Cat.3; R62 | X |  |  |
| 1.2.-Propylene glycol-dimethyl ether | 7778-85-0 | F; R11-19 | X |  |  |
| TXIB | 6846-50-0 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified.. | X |  |  |
| Diethylene glycol monoethyl ether | 111-90-0 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified.. | X |  |  |
| Dipropylene glycol dimethyl ether | 63019-84-1 89399-28-0 111109-77-4 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified.. | X |  |  |
| Aldehydes |  |  |  |  |  |
| Acrolein | 107-02-8 | F; R11 / T+; R26 / T; R24/25 / C; R34 / N; R50 |  | X |  |
| Acetaldehyde | 75-07-0 | F+; R12 / Carc.Cat.3; R40 / Xi; R36/37 |  | X |  |
| Propanal | 123-38-6 | F; R11 / Xi; R36/37/38 |  | X |  |
| Butyraldehyde | 123-72-8 | F; R11 |  | X |  |
| Pentanal | 110-62-3 | F; R11 / Xi; R38 / R41 | x | X |  |
| Hexanal | 66-25-1 | R10 / Xi; R36/38 | X |  |  |
| Heptanal | 111-71-7 | R10 / Xi; R38 | X |  |  |
| 2-Ethyl-hexanal | 123-05-7 | R10 / Xi; R 36/37/38 | X |  |  |
| Octanal | 124-13-0 | Xi / R38 | X |  |  |
| Nonanal | 124-19-6 | Xi; R36/38 | X |  |  |
| Decanal | 112-31-2 | Xi / R38 | X |  |  |
| 2-Butenal (crotonaldehyde, cis-trans-mix) | 4170-30-3 123-73-9 15798-64-8 | F; R11 / Muta.Cat.3; R68 / T+; R26 / T; R24/25 / Xn; R48/22 / Xi; R37/38-41 / N; R50 | x | X |  |
|  |  |  | x | X |  |
|  |  |  | X |  |  |
| 2-Pentenal | 1576-87-0 764-39-6 31424-04-1 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| 2-Hexenal | 16635-54-4 6728-26-3 505-57-7 1335-39-3 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| 2-Heptenal | 2463-63-0 18829-55-5 29381-66-6 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| 2-Octenal | 2363-89-5 25447-69-2 20664-46-4 2548-87-0 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| 2-Nonenal | 2463-53-8 30551-15-6 18829-56-6 60784-31-8 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| 2-Decenal | 3913-71-1 / 2497-25-8 / 3913-81-3 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| 2-Undecenal | 2463-77-6 / 53448-07-0 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| Furfural | 98-01-1 | Carc.Cat.3; R40 / T; R23/25 / Xn; R21 / Xi; R36/37/38 | x | X |  |
| Glutardialdehyde | 111-30-8 | T; R23/25 / C; R34 / R42/43 / N; R50 | x | X |  |
| Benzaldehyde | 100-52-7 | Xn; R22 | x | X |  |
| Ketones |  |  |  |  |  |
| Acetone | 67-64-1 | F; R11 / Xi; R36 / R66 / R67 |  | X |  |
| Ethylmethylketone | 78-93-3 | F; R11 / Xi; R36 / R66 / R67 | x | X |  |
| 3-Methylbutanone-2 | 563-80-4 | F; R11 | X |  |  |
| Methylisobutylketone | 108-10-1 | F; R11 / Xn; R20 / Xi; R36/37 / R66 | x | X |  |
| Di-isobutyl ketone | 108-83-8 | R10 / Xi; R37 |  | X |  |
| Cyclopentanone | 120-92-3 | R10 / Xi; R36/38 | X |  |  |
| Cyclohexanone | 108-94-1 | R10 / Xn; R20 | x | x | X |
| Hexan-2-one | 591-78-6 | R10 / Repr.Cat.3; R62 / T; R48/23 / R67 |  | X |  |
| 2-Methylcyclopentanone | 1120-72-5 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| 2-Methylcyclohexanone | 583-60-8 | R10 / Xn; R20 | x | X |  |
| Acetophenone | 98-86-2 | Xn; R22 / Xi; R36 | x | X |  |
| 1-Hydroxyacetone (2 Propanone, 1-hydrocx-) | 116-09-6 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| Heptane-2-one | 110-43-0 | R10 / Xn; R20/22 |  | X |  |
| Butyl ethyl ketone | 106-35-4 | R10 / Xn; R20 / Xi; R36 |  | X |  |
| 3,5,5-Trimethylcyclohex-2-enone | 78-59-1 | Carc.Cat.3; R40 / Xn; R21/22 / Xi; R36/37 |  | X |  |
| Camphor | 76-22-2 | F; R11 / Xi; R 36/37/38 |  | X |  |
| 2-Chloroacetophenone | 532-27-4 | T; R 25 / R 36/37/38 / R 42/43 |  | X |  |
| Chloroacetone | 78-95-5 | R 10 / T; R 23/24/25 / R 36/37/38 / N; R 50/53 |  | X |  |
| Mesityl oxide | 141-79-7 | R10 / Xn; R20/21/22 |  | X |  |
| 5-Methylhexan-2-one | 110-12-3 | R10 / Xn; R20 |  | X |  |
| 5-Methylheptan-3-one | 541-85-5 | R10 / Xi; R36/37 |  | X |  |
| Methyl propyl ketone | 107-87-9 | F; R 11 / Xn; R 22 / R 36/37/38 |  | X |  |
| methyl vinyl ketone | 78-94-4 | F; R 11 / R 25 / T+; R 26/27 / R 34 / R 43 / N; R 50/53 |  | X |  |
| Acids |  |  |  |  |  |
| Acrylic acid | 79-10-7 | R10 / Xn; R20/21/22 / C; R35 / N; R50 |  | x |  |
| Acetic acid | 64-19-7 | R10 / C; R35 | x | X |  |
| Propionic acid | 79-09-4 | C; R34 | x | X |  |
| Isobutyric acid | 79-31-2 | Xn; R21/22 | X |  |  |
| Butyric acid | 107-92-6 | C; R34 | X |  |  |
| Pivalic acid | 75-98-9 | Xn; R 21/22 / R38 / R41 | X |  |  |
| n-Valeric acid | 109-52-4 | C; R34 / R52/53 | X |  |  |
| n-Caproic acid | 142-62-1 | C; R34 | X |  |  |
| n-Heptanoic acid | 111-14-8 | C; R34 | X |  |  |
| n-Octanoic acid | 124-07-2 | C; R34 | X |  |  |
| 2-Ethylhexane acid | 149-57-5 | Repr.Cat.3; Xn; R63 | X |  |  |
| 2,4-D | 94-75-7 | Xn; R22 / Xi; R37-41 / R43 / R52/53 |  | X |  |
| Chloroacetic acid | 79-11-8 | T; R23/24/25 / C; R34 / N; R50 |  | X |  |
| Methacrylic acid | 79-41-4 | Xn; R21/22 / C; R35 |  | X |  |
| Formic Acid | 64-18-6 | C; R35 |  | X |  |
| Perchloromethylmercaptan | 594-42-3 | R 21 / R 25 / T+; R 26 / R 34 |  | X |  |
| Thioglycolic acid | 68-11-1 | T; R23/24/25 / C; R34 |  | X |  |
| Esters and Lactones |  |  |  |  |  |
| Methyl acetate | 79-20-9 | F; R11 / Xi; R36 / R66 / R67 |  | X |  |
| Ethyl acetate | 141-78-6 | F; R11 / Xi; R36 / R66 / R67 |  | x | X |
| Vinyl acetate | 108-05-4 | F; R11 |  | x | X |
| Methyl formate | 107-31-3 | F+; R12 / Xn; R20/22 / Xi; R36/37 |  | X |  |
| Ethyl formate | 109-94-4 | F; R11 / Xn; R20/22 / Xi; R36/37 |  | X |  |
| Isopropyl acetate | 108-21-4 | F; R11 / Xi; R36 / R66 / R67 | x | X |  |
| Propyl acetate | 109-60-4 | F; R11 / Xi; R36 / R66 / R67 | x | X |  |
| 2-Methoxy-1-methylethyl acetate | 108-65-6 | R10 | x | X |  |
| n-Butyl formiate | 592-84-7 | F; R11 / Xi; R36/37 | X |  |  |
| Methyl methacrylate | 80-62-6 | F; R11 / Xi; R37/38 / R43 | x | X |  |
| Ethyl methacrylate | 97-63-2 | F; R11 / Xi; R36/37/38 / R43 |  | X |  |
| Other methacrylates |  |  |  |  |  |
| Isobutyl acetate | 110-19-0 | F; R11 / R66 | x | X |  |
| 1-Butyl acetate | 123-86-4 | R10 / R66 / R67 | x | x | X |
| 2-Butyl acetate | 105-46-4 | F; R11 / R66 |  | X |  |
| Tert-Butyl acetate | 540-88-5 | F; R11 / R66 |  | X |  |
| 2-Ethylhexyl acetate | 103-09-3 | Xi ; R38 | x | X |  |
| Methyl acrylate | 96-33-3 | F; R11 / Xn; R20/21/22 / Xi; R36/37/38 / R43 | x | X |  |
| Ethyl acrylate | 140-88-5 | F; R11 / Xn; R20/21/22 / Xi; R36/37/38 / R43 | x | X |  |
| n-Butyl acrylate | 141-32-2 | R10 / Xi; R36/37/38 / R43 | x | X |  |
| Isobutyl acrylate | 106-63-8 | R10 / Xn; R20/21 / Xi; R38 / R43 |  | X |  |
| Tert-Butyl acrylate | 1663-39-4 | F; R11 / Xn; R20/21/22 / Xi; R37/38 / R43 / R52/53 |  | X |  |
| 2-Ethylhexyl acrylate | 103-11-7 | Xi; R37/38 / R43 | X |  |  |
| Other acrylates (acrylic acid ester) |  |  | x |  |  |
| Dimethyl adipate | 627-93-0 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| Dibutyl fumarate | 105-75-9 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| Dimethyl succinate | 106-65-0 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| Dimethyl glutarate | 1119-40-0 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| Hexamethylene diacrylate | 13048-33-4 | Xi; R36/38 / R43 | X |  |  |
| Maleic acid dibutylester | 105-76-0 | Xi; R43 / N; R 51/53 | X |  |  |
| Tetramethyl orthosilicate | 681-84-5 | R 10 / T+; R 26 / R 37/38 / R 41 |  | X |  |
| Butyrolactone | 96-48-0 | Xn; R22 / R36 | X |  |  |
| Demeton | 8065-48-3 | T+; R27/28 / N; R50 |  | X |  |
| Diazinon | 333-41-5 | Xn; R22 / N; R50/53 |  | X |  |
| Ethyl lactate | 97-64-3, 687-47-8 | R10 / Xi; R37-41 |  | X |  |
| 1,3-Dimethylbutyl acetate | 108-84-9 | R10 / Xi; R36/37 |  | X |  |
| Malathion | 121-75-5 | Xn; R22 / N; R50/53 |  | X |  |
| Methyl parathion | 298-00-0 | R5 / R10 / T+; R26/28 / T; R24 / Xn; R48/22 / N; R50/53 |  | X |  |
| Mevinphos | 7786-34-7 | T+; R27/28 / N; R50/53 |  | X |  |
| Naled | 300-76-5 | Xn; R21/22 / Xi; R36/38 / N; R50 |  | X |  |
| Pentyl acetates | 626-38-0, 624-41-9, 84145-37-9, 620-11-1, 628-63-7, 123-92-2 | R10 / R66 |  | X |  |
| Pyrethrins including cinerins | 8003-34-7 | Xn; R20/21/22 / N; R50/53 |  | X |  |
| Ethyl-2-cyanoacrylate | 7085-85-0 | Xi; R36/37/38 |  | X |  |
| Mecrilate | 137-05-3 | Xi; R36/37/38 |  | X |  |
| Triphenyl phosphate | 115-86-6 | N / R50/53 |  | X |  |
| O,o,o-Tricresylphosphate | 78-30-8 | T; R39/23/24/25 / N; R51/53 |  | X |  |
| Trimethyl phosphite | 121-45-9 | T; R46 / R10 / R22 / R36/37/38 |  | X |  |
| n-Butyl lactate | 138-22-7 | Xi; R36/37/38 |  | X |  |
| Dibutyl phthalate | 84-74-2 | Repr.Cat.2; R61 / Repr.Cat.3; R62 / N; R50 |  |  | X |
| Sulfotep | 3689-24-5 | T+; R27/28 / N; R50/53 |  | X |  |
| Chlorinated hydrocarbons |  |  |  |  |  |
| Tetrachloroethene | 127-18-4 | Carc.Cat.3; R40 / N; R51/53 | x | X |  |
| Methyl chloride | 74-87-3 | F+; R12 / Carc.Cat.3; R40 / Xn; R48/20 |  | X |  |
| Chloroethane | 75-00-3 | F+; R12 / Carc.Cat.3; R40 / R52/53 |  | X |  |
| 1,1,2,2-Tetrachloroethane | 79-34-5 | T+; R26/27 / N; R51/53 |  | X |  |
| 1,1,2-Trichloroethane | 79-00-5 | Carc.Cat.3; R40 / Xn; R20/21/22 / R66 |  | x | X |
| 1,1-Dichloroethylene | 75-35-4 | F+; R12 / Carc.Cat.3; R40 / Xn; R20 |  | X |  |
| 2,3,4,5-Tetrachlorophenol | 4901-51-3 | T: R 25 / R 37/38 / R 41 |  | x | X |
| 2,3,4,6-Tetrachlorophenol | 58-90-2 | T; R25 / Xi; R36/38 / N; R50/53 |  | x | X |
| 1,2-Dichlorobenzene | 95-50-1 | Xn; R22 / Xi; R36/37/38 / N; R50/53 |  | x | X |
| 1,4-Dichlorobenzene | 106-46-7 | Xi; R36 / Carc.Cat.3; R40 / N; R50/53 |  | x | X |
| 1,2,3-Trichlorobenzene | 87-61-6 | Xn; R22 / N; R50/53 |  | X |  |
| 1,2,4-Trichlorobenzene | 120-82-1 | Xn; R22 / Xi; R38 / N; R50/53 |  | X |  |
| 1,3,5-Trichlorobenzene | 108-70-3 | Xn: R22 / R38 |  | X |  |
| (Z)-1,2-Dichloroethylene | 156-59-2 | F; R11 / Xn; R20 / R52/53 |  | X |  |
| 1,1-Dichloroethane | 75-34-3 | F; R11 / Xn; R22 / Xi; R36/37 / R52/53 |  | X |  |
| 1,2-Dichloroethylene | 540-59-0 | F; R11 / Xn; R20 / R52/53 |  | X |  |
| trans-1,2-Dichloroethylene | 156-60-5 | F; R11 / Xn; R20 / R52/53 |  | X |  |
| Methylene chloride | 75-09-2 | Carc.Cat.3; R40 |  | X |  |
| 1,1-Dichloro-1-nitroethane | 594-72-9 | T; R23/24/25 |  | X |  |
| 1,2-Dichloropropane | 78-87-5 | F; R11 / Xn; R20/22 |  | X |  |
| 1,2-Dichloro-1,1,2,2-tetrafluoroethane | 76-14-2 | N; R59 |  | X |  |
| Pentachloroethane | 76-01-7 | Carc.Cat.3; R40 / T; R48/23 / N; R51/53 |  | X |  |
| Dichlorvos | 62-73-7 | T+; R26 / T; R24/25 / R43 / N; R50 |  | X |  |
| Dicrotophos | 141-66-2 | T+; R28 / T; R24 / N; R50/53 |  | X |  |
| Ethylene chlorohydrin | 107-07-3 | T+; R26/27/28 |  | X |  |
| Aldrin | 309-00-2 | T; R24/25-48/24/25 / Carc.Cat.3; R40 / N; R50/53 |  | X |  |
| Hexachloroethane | 67-72-1 | Carc.Cat.3; R40 |  | X |  |
| Hexachlorocyclopentadiene | 77-47-4 | T+; R26 / T; R24 / Xn; R22 / C; R34 / N; R50/53 |  | X |  |
| Chloroacetaldehyde | 107-20-0 | Carc.Cat.3; R40 / T+; R26 / T; R24/25 / C; R34 / N; R50 |  | X |  |
| 2-Chloroacetophenone | 532-27-4 | T; R 25 / R 36/37/38 / R 42/43 |  | X |  |
| Trichloronitromethane | 76-06-2 | Xn; R22 / T+; R26 / Xi; R36/37/38 |  | X |  |
| Chloroform | 67-66-3 | Xn; R22-48/20/22 / Xi; R38 / Carc.Cat.3; R40 |  | X |  |
| 1-Chloro-4-nitrobenzene | 100-00-5 | Carc.Cat.3; R40 / Muta.Cat.3; R68 / T; R23/24/25 / Xn; R48/20/21/22 / N; R51/53 |  | X |  |
| Cyanuric chloride | 108-77-0 | T+; R26 / Xn; R22 / C; R34 / R43 / R14 |  | X |  |
| Tetrachlorophthalic anhydride | 117-08-8 | Xi; R41 / R42/43 / N; R50/53 |  | X |  |
| Trichlorfone | 52-68-6 | Xn; R22 / R43 / N; R50/53 |  | X |  |
| Trichlorovinylsilane | 75-94-5 | F; R 11 / R 14 / R 20/21/22 / C; R 34 |  | X |  |
| Heptachlor | 76-44-8 | T; R24/25 / Carc.Cat.3; R40 / R33 / N; R50/53 |  | X |  |
| Further VOC |  |  |  |  |  |
| 1.4-Dioxan | 123-91-1 | F; R11-19 / Carc.Cat.3; R40 / Xi; R36/37 / R66 | x | X |  |
| Caprolactam | 105-60-2 | Xn; R20/22 / Xi; R36/37/38 | x | X |  |
| N-methyl-2-pyrrolidon | 872-50-4 | Xi; R36/38 | x | X |  |
| Octamethylcyclotetra-siloxane | 556-67-2 | Repr.Cat.3; Xn; R62 / R53 | X |  |  |
| Hexamethylenetetramine | 100-97-0 | F; R11 / R42/43 | X |  |  |
| 2-Butanonoxime | 96-29-7 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| Tributyl phosphate | 126-73-8 | Carc.Cat. 3; R40 / Xn; R22 / Xi; R38 | x | X |  |
| Triethyl phosphate | 78-40-0 | Xn; R22 | X |  |  |
| 5-Chloro-2-methyl-2H-isothiazol-3-one (CIT) | 26172-554 | Not classified in Annex 1 of the Directive 67/548/EEC. Substances not listed must be self classified. | X |  |  |
| 2-Methyl-4-isothiazolin-3-one | 2682-20-4 | R 22 / T; R 23 / R 34 / R 43 / N, R 50 | X |  |  |
| Triethylamine | 121-44-8 | F; R11 / Xn; R20/21/22 / C; R35 | X |  |  |
| Acetonitrile | 75-05-8 | F; R11 / Xn; R20/21/22 / Xi; R36 |  | X |  |
| Allyl glycidyl ether | 106-92-3 | R10 / Carc.Cat.3; R40 / Muta.Cat.3; R68 / Repr.Cat.3; R62 / Xn; R20/22 / Xi; R37/38-41 / R43 / R52/53 |  | X |  |
| Allyl chloride | 107-05-1 | F; R11 / Carc.Cat.3; R40 / Muta.Cat.3; R68 / Xn; R20/21/22-48/20 / Xi; R36/37/38 / N; R50 |  | X |  |
| 2-Aminoethanol | 141-43-5 | Xn; R20/21/22 / C; R34 |  | X |  |
| 2-Aminopyridine | 504-29-0 | T ; R23/24/25 / N; R51/53 |  | X |  |
| tert-Amylmethylether | 994-05-8 | F ; R11 / Xn; R22 |  | X |  |
| Atrazine | 1912-24-9 | Xn; R48/22 / R43 / N; R50-53 |  | X |  |
| Bis(2-chloroethyl) ether | 111-44-4 | R10 / Carc.Cat.3; R40 / T+; R26/27/28 |  | X |  |
| Bromoform | 75-25-2 | T; R23 / Xi; R36/38 / N; R51/53 |  | X |  |
| Butylamine | 109-73-9 | F; R11 / Xn; R20/21/22 / C; R35 |  | X |  |
| 1-Bromopropane | 106-94-5 | F; R11 / Repr.Cat.2; R60 / Repr.Cat.3; R63 / Xn; R48/20 Xi; R36/37/38 / R67 |  | X |  |
| 2-Bromopropane | 75-26-3 | F; R11 / Repr.Cat.1; R60 / Xn; R48/20 / R66 |  | X |  |
| 1-Butanethiol | 109-79-5 | F; R11 / Xn; R20/22 |  | X |  |
| Sec-Butylamine | 13952-84-6 | F; R11 / Xn; R20/22 / C; R35 / N; R50 |  | X |  |
| Dibromodifluoromethane | 75-61-6 | Xi; R 36/37/38 |  | X |  |
| 2-Diethylaminoethanol | 100-37-8 | R10 / Xn; R20/21/22 / C; R34 |  | X |  |
| Diethyl ether | 60-29-7 | F+; R12 / R19 / Xn; R22 / R66 / R67 |  | X |  |
| Diphenylamine | 122-39-4 | T; R23/24/25 / R33 / N; R50/53 |  | X |  |
| Diglycidyl ether | 2238-07-5 | Carc.Cat.3; R 40 / R 19 / R 21/22 / T; R 23 / R 34 |  | X |  |
| Dimethylamine | 124-40-3 | F+; R12 / Xn; R20 / Xi; R37/38-41 |  | X |  |
| Ethyldimethylamine | 598-56-1 | F+; R12 / Xn; R20/22 / C; R34 |  | X |  |
| Dimethyl formamide | 68-12-2 | Repr.Cat.2; R61 / Xn; R20/21 / Xi; R36 |  | X |  |
| Dinitrobenzene | 25154-54-5 | T+; R26/27/28 / R33 / N; R50/53 |  | X |  |
| 4,6-Dinitro-o-cresol | 534-52-1 | Muta.Cat.3; R68 / T+; R26/27/28 / Xi; R38-41 / R43 / R44 / N; R50/53 |  | X |  |
| Disulfiram | 97-77-8 | Xn; R22-48/22 / R43 / N; R50/53 |  | X |  |
| Acetic anhydride | 108-24-7 | R10 / Xn; R20/22 / C; R34 |  | X |  |
| 2,2'-Iminodiethanol | 111-42-2 | Xn; R22-48/22 / Xi; R38-41 |  | X |  |
| Methanethiol | 74-93-1 | F+; R12 / T; R23 / N; R50/53 |  | X |  |
| Ethanethiol | 75-08-1 | F; R11 / Xn; R20 / N; R50-53 |  | X |  |
| Formaldehydedimethylacetal | 109-87-5 | F; R11 / R19 / Xi; R36 |  | X |  |
| Formamide | 75-12-7 | Repr.Cat.2; R61 |  | X |  |
| Bromoethane | 74-96-4 | F; R11 / Carc.Cat.3; R40 / Xn; R20/22 |  | X |  |
| 4-Ethylmorpholine | 100-74-3 | R 10 / R 21/22 / C ; R 34 |  | X |  |
| Phenothiazine | 92-84-2 | Xi; R36/37/38 |  | X |  |
| Carbon tetrabromide | 558-13-4 | Xn; R 20/21/22 |  | X |  |
| Iodoform | 75-47-8 | Xn; R 20/21/22 / R 36/37/38 |  | X |  |
| Methyl iodide | 74-88-4 | Carc.Cat.3; R40 / Xn; R21 / T; R23/25 / Xi; R37/38 |  | X |  |
| Maleic anhydride | 108-31-6 | Xn; R22 / C; R34 / R42/43 |  | x | X |
| Methacrylonitrile | 126-98-7 | F; R11 / T; R23/24/25 / R43 |  | X |  |
| Morpholine | 110-91-8 | R10 / Xn; R20/21/22 / C; R34 |  | X |  |
| Piperazine | 110-85-0 | C; R34 / R42/43 / R52/53 |  | X |  |
| Pyridine | 110-86-1 | F; R11 / Xn; R20/21/22 |  | X |  |
| Rotenone | 83-79-4 | T; R25 / Xi; R36/37/38 / N; R50/53 |  | X |  |
| Tetrahydrofuran | 109-99-9 | F; R11-19 / Xi; R36/37 |  | X |  |
| 3a,4,7,7a-Tetrahydro-4,7-methanoindene | 77-73-6 | F; R11 / Xn; R20/22 / Xi; R36/37/38 / N; R51/53 |  | X |  |
| 1,2,3,6-Tetrahydromethyl-phthalic acid anhydride | 26590-20-5 | Xi; R41 / Xn; R42/43 |  | X |  |
| 1-Vinyl-2-pyrrolidone | 88-12-0 | Carc.Cat.3; R40 / Xn; R20/21/22-48/20 / Xi; R37-41 |  | X |  |
| Vinylcyclohexane diepoxide | 106-87-6 | T; R23/24/25 / Xn; R68 |  | X |  |

B-3. Carcinogenic VOC of EU-categories 1 and 2 extracted from Annex VI Part 3 of the Regulation (EC) Nr. 1272/2008 (no claim of completeness can be made)

All listed volatile carcinogens are regulated under 2009-167-D (sum of all volatile carcinogens). Substances for which substance specific indoor air limit values or product emission limit values have been set additionally at national level are highlighted (regulations 2009-34-F, 2007-372-FIN, 1996-PL). All listed substances are potential future SVHC under REACH.

Table B-3.1 Carcinogenic VVOC (boiling point below 68 °C)

|  | Name | CAS-Nr. | Carc. cat |
| --- | --- | --- | --- |
|  | Bromoethane | 74-96-4 | 2 |
|  | Bromoethylen | 593-60-2 | 2 |
|  | 1,3-Butadiene | 106-99-0 | 1 |
|  | n-Butane (includes >= 0,1 % Butadiene) | 106-97-8 | 1 |
|  | Chloroprene | 126-99-8 | 2 |
|  | Chlorofluoromethane | 593-70-4 | 2 |
|  | Chlorodimethyl ether | 107-30-2 | 1 |
|  | Diazomethane | 334-88-3 | 2 |
|  | N,N-Dimethylhydrazine | 57-14-7 | 2 |
|  | Dimethyl sulfate | 77-78-1 | 2 |
|  | 1,2-Epoxybutan | 106-88-7 | 2 |
|  | Ethyleneimine | 151-56-4 | 2 |
|  | Ethylene oxide | 75-21-8 | 2 |
|  | Furan | 110-00-9 | 2 |
|  | Isobutyl nitrite | 542-56-3 | 2 |
|  | Isoprene | 78-79-5 | 2 |
|  | 2-Methylaziridine | 75-55-8 | 2 |
|  | Propylene oxide | 75-56-9 | 2 |
|  | Chloroform | 67-66-3 | 2 |
|  | Vinyl chloride | 75-01-4 | 1 |

Table B-3.2 Carcinogenic VOC (boiling point between 68 and 250 °C)

|  | Name | CAS-Nr. | Carc. cat |
| --- | --- | --- | --- |
|  | Acrylamide | 79-06-1 | 2 |
|  | Acrylonitrile | 107-13-1 | 2 |
|  | Allyl glycidyl ether | 106-92-3 | 2 |
|  | Benzene | 71-43-2 | 1 |
|  | 2,2'-Bioxirane | 1464-53-5 | 2 |
|  | Bis (chloromethyl) ether | 542-88-1 | 1 |
|  | Resorcinol diglycidyl ether | 101-90-6 | 2 |
|  | 4-Chloroaniline | 106-47-8 | 2 |
|  | Epichlorhydrine | 106-89-8 | 2 |
|  | (R)-(-)-Epichlorohydrine | 51594-55-9 | 2 |
|  | 4-Chloro-2-methylaniline | 95-69-2 | 1 |
|  | Benzyl chloride | 100-44-7 | 2 |
|  | Chromyl chloride | 14977-61-8 | 2 |
|  | 1,2-Dibromo-3-chloropropane | 96-12-8 | 2 |
|  | 1,2-Dibromoethane | 106-93-4 | 2 |
|  | 1,4-Dichlorobut-2-ene | 764-41-0 | 2 |
|  | Bis(2-chloroethyl) sulfide | 505-60-2 | 1 |
|  | Ethylene dichloride | 107-06-2 | 2 |
|  | 1,3-Dichloro-2-propanol | 96-23-1 | 2 |
|  | 1,3-Dichloropropene, isomers | 542-75-6 | 2 |
|  | Dimethylcarbamoyl chloride | 79-44-7 | 2 |
|  | N,N'-Dimethylhydrazine; 1,2-Dimethylhydrazine | 540-73-8 | 2 |
|  | Vinylcyclohexane diepoxide | 106-87-6 | 2 |
|  | Hexamethylphosphoric triamide | 680-31-9 | 2 |
|  | 2-Methoxyaniline | 90-04-0 | 2 |
|  | 6-Methoxy-m-toluidine | 120-71-8 | 2 |
|  | Methyl azoxy methyl acetate | 592-62-1 | 2 |
|  | N-Methylbis(2-chloroethyl)amine | 51-75-2 | 1 |
|  | 4,4-Methylenedi-o-toluidine | 838-88-0 | 2 |
|  | Morpholine-4-carbonyl chloride | 15159-40-7 | 2 |
|  | 2-Nitropropane | 79-46-9 | 2 |
|  | N-Nitrosodibutylamine | 924-16-3 | 2 |
|  | N-Nitrosodiethylamine | 55-18-5 | 2 |
|  | Nitrosodipropylamine | 621-64-7 | 2 |
|  | N-Nitrosodiisopropylamine | 601-77-4 | 2 |
|  | N-Ethyl-N-nitrosoaniline | 612-64-6 | 2 |
|  | 2,2'-(Nitrosoimino)bisethanol | 1116-54-7 | 2 |
|  | N-Methyl-N-nitroso-ethylamine | 10595-95-6 | 2 |
|  | N-Nitrosomorpholine | 59-89-2 | 2 |
|  | N-Nitrosopiperidine | 100-75-4 | 2 |
|  | N-Nitrosopyrrolidine | 930-55-2 | 2 |
|  | 2-Nitrotoluene | 88-72-2 | 2 |
|  | Phenyl glycidyl ether | 122-60-1 | 2 |
|  | 1,3-Propansulton | 1120-71-4 | 2 |
|  | 5-Allyl-1,3-benzodioxole | 94-59-7 | 2 |
|  | Styrene oxide | 96-09-3 | 2 |
|  | Sulfallate | 95-06-7 | 2 |
|  | 4-Chlorobenzotrichloride | 5216-25-1 | 2 |
|  | Tetranitromethane | 509-14-8 | 2 |
|  | o-Toluidine | 95-53-4 | 2 |
|  | ,3,4-Trichlorobut-1-ene | 2431-50-7 | 2 |
|  | Trichloroethylene | 79-01-6 | 2 |
|  | 1,2,3-Trichloropropane | 96-18-4 | 2 |
|  | Benzotrichloride | 98-07-7 | 2 |
|  | 2,4,5-Trimethylanilin | 137-17-7 | 2 |
|  | Urethane | 51-79-6 | 2 |

B-3.3 Carcinogenic SVOC (boiling point between 250 and 400 °C)

|  | Name | CAS-Nr. | Carc. Cat |
| --- | --- | --- | --- |
|  | 4-Aminoazobenzene | 60-09-3 | 2 |
|  | 4-Aminobiphenyl | 92-67-1 | 1 |
|  | Azobenzene | 103-33-3 | 2 |
|  | 4,4'-Bi-o-toluidine | 119-93-7 | 2 |
|  | 4,4'-Diaminodiphenylmethane (MDA) | 101-77-9 | 2 |
|  | 4-Methyl-m-phenylenediamine | 95-80-7 | 2 |
|  | Hexachlorobenzene | 118-74-1 | 2 |
|  | 4-Methyl-m-phenylenediamine | 95-80-7 | 2 |
|  | 2-Naphthylamine | 91-59-8 | 1 |
|  | 2-Nitroanisole | 91-23-6 | 2 |
|  | 4-Nitrobiphenyle | 92-93-3 | 2 |
|  | 2-Nitronaphtalene | 581-89-5 | 2 |
|  | Methylenebisdimethylaniline | 101-61-1 | 2 |

Annex C

List of consulted notifications (see http://ec.europa.eu/enterprise/tris/)

| Notification No. | Title in original language | Title in English |
| --- | --- | --- |
| 1985-26-D | Gefahrstoffverordnung | Order on protection from hazardous substances (not available in database TRIS as notifications from 1980s are not included) |
| 1987-0125-F | Décret n°88-683 du 6 mai 1988 relatif à l'utilisation des mousses urée-formol dans les locaux à usage d'habitation ou destinés à une occupation humaine permanente ou semi-permanente Arrêté du 6 mai 1988 relatif à la teneur maximale en formaldéhyde provenant de l'injection des mousses urée-formol dans les locaux à usage d'habitation ou destinés à une occupation permanente ou semi-permanente | Décret 88-683 of 06/05/1988 on the use of urea-formaldehyde foams in buildings intended for permanent or semi-permanent human occupation Arrêté of 06/05/1988 fixing a maximum content of formaldehyde coming from urea-formaldehyde foams in buildings intended for a permanent or semi-permanent human occupation. |
| 1993-141-D | Verordnung zur Ausweitung der Verbote von Stoffen, Zubereitungen und Erzeugnissen, die bestimmte polyhalogenierte Dibenzo-p-dioxine (phdd) und bestimmte polyhalogenierte Dibenzofurane (phdf) enthalten. | Prohibition of materials, preparations and products containing certain polyhalogenated dibenzo-p-dioxins (phdds) or certain polyhalogenated dibenzofuranes (phdfs) (extension of applicability) order. |
| 1995-188-DK | Bygningsreglement | Building regulations |
| 1995-213-A | Verordnung des Bundesministers für Umwelt über Verbote und Beschränkungen von organischen Lösungsmitteln (Lösungsmittelverordnung 1995 - LMVO 1995) | Order of the Federal Minister for the Environment on bans on and restrictions of organic solvents (Solvents Order 1995 - German designation LMVO 1995) |
| 1996-183-D | Muster-Liste der Technischen Baubestimmungen - Fassung April 1996 - einschließlich Muster-Einführungserlaß | Specimen introductory order for the sample list of Technical Building Provisions |
| 1997/346/NL | Ontwerp-Regeling Bouwbesluit materialen | Draft Regulation Building Decree on Materials |
| 1997-527-DK | Bygningsreglement for småhuse | Building regulations for small houses |
| 1998/57/NL | Ontwerp-Regeling Bouwbesluit materialen 1998 | Draft Regulation under the Buildings Decree on Materials 1998 |
| 1998-156-D | Verordnung zur Änderung chemikalienrechtlicher Verordnungen | Order amending the Orders on chemical law |
| 1998-9034-N | Forskrift om forbud mot bruk av akrylamidbaserte injeksjonsmidler til tetting av vannlekkasjer i anleggsvirksomhet | A proposal for regulations relating to a permanent prohibition against the use of acryl amide based grouting agents for the sealing against water leakages in connection with construction work. (incorporated into 2002-9039-N) |
| 1999-263-A | RVS 8S.01.31 Technische Vertragsbedingungen Wiederverwendbare Baustoffe: Asphaltgranulat | Guidelines and Regulations for Highway Construction [German designation: RVS] 8S.01.31 Technical terms of contract Reusable construction materials Asphalt granulate |
| 2001-121-DK | Bekendtgørelse om regulering af visse industrielle drivhusgasser: BEK nr 552 af 02/07/2002 (Gældende) | Order regulating certain industrial greenhouse gases |
| 2001-200-A | Gütevorschriften für elastische Bodenbeläge | Quality regulations for elastic flooring materials |
| 2001-450-D | Verordnung über die Entsorgung von Altholz | Order on the disposal of waste wood |
| 2002-271-FIN | Ympäristöministeriön asetus rakennusten sisäilmastosta ja ilmanvaihdosta, Suomen rakentamismääräyskokoelma, osa D2, määräykset ja ohjeet | Ministry of the Environment decree on indoor air and ventilation in buildings, the Finnish Building Regulations, Part D2, regulations and instructions |
| 2002-9039-N | Forskrift om begrensning i bruk av helse- og miljøfarlige kjemikalier og andre produkter (produktforskriften) | Regulations relating to restrictions on certain dangerous chemicals and products |
| 2004-294-NL | Ontwerpbesluit houdende regels betreffende asbest en asbesthoudende producten (Productenbesluit asbest) | Draft Decree regulating asbestos and products containing asbestos (Asbestos Products Decree) |
| 2004-410-NL | Ontwerpregeling houdende regels met betrekking tot het bepalen van de concentratie asbest in producten (Productenregeling asbest). | Draft regulation on determining the concentration of asbestos in products (Asbestos Products Regulation). |
| 2004-71-D | Technische Lieferbedingungen für Gesteinskörnungen im Straßenbau, TL Gestein-StB 04 | Technical Terms of Delivery for aggregates used in road construction [German designation: TL Gestein-StB 04] |
| 2005-255-D | Grundsätze zur gesundheitlichen Bewertung von Bauprodukten in Innenräumen, Stand April 2005 | Principles for the health assessment of construction products used in interiors, as at April 2005 |
| 2005-283-NL | Regeling tot wijziging van de Uitvoeringsregeling Bouwstoffenbesluit bodem- en oppervlaktewaterenbescherming | Regulation amending the Regulation implementing the Building Materials (Soil and Surface Water Protection) Decree; |
| 2005-485-A | Verordnung des Bundesministers für Land- und Forstwirtschaft, Umwelt und Wasserwirtschaft, mit der die Verordnung über Verbote und Beschränkungen teilfluorierter und vollfluorierter Kohlenwasserstoffe sowie von Schwefelhexafluorid (HFKW-FKW-SF6-V) geändert wird | Order issued by the Federal Minister for Agriculture, Forestry, Environment and Water Management amending the Order governing prohibitions and restrictions on partially fluorinated and fully fluorinated hydrocarbons and sulphur hexafluoride (HFC-CFC-SF6-Order). |
| 2005-649-S (withdrawn from 1st June 2008) | Förordning om ändring i förordningen (1998:944) om förbud m.m. i vissa fall i samband med hantering, införsel och utförsel av kemiska produkter | Order amending the Order (1998:994) imposing bans etc. in certain cases in connection with the handling, import and export of chemical products |
| 2005-735-FIN | Valtioneuvoston asetus eräiden jätteiden hyödyntämisestä maarakentamisessa | Council of State Decree concerning the recovery of certain wastes in earth construction |
| 2005-9020-N | Forskrift om begrensning i bruk av helse- og miljøfarlige kjemikalier og andre produkter | Regulations relating to restrictions on the use of chemicals dangerous to health and environment and other products |
| 2006-223-E | DECRETO SOBRE VALORIZACIÓN DE ESCORIAS PROCEDENTES DE LA FABRICACIÓN DE ACERO EN HORNO DE ARCO ELÉCTRICO | Decree on the recovery of slag produced from the manufacture of steel in electric arc furnaces |
| 2006-90-D | Grundsätze zur Bewertung der Auswirkungen von Bauprodukten auf Boden und Grundwasser | Principles for assessing the effects of construction products on soil and groundwater |
| 2006-496-NL | Besluit van 22 november 2007, houdende regels inzake de kwaliteit van de bodem (Besluit bodemkwaliteit) | Soil Quality Decree |
| 2006-516-D | Eisenbahnspezifische Bauregellisten (EBRL) und Eisenbahnspezifische Ergänzungen und Anlagen zu den Bauregellisten A, B und der Liste C des DIBt (Ausgabe 2005/1) | Railway-specific Building Regulation Lists (German designation EBRL) and railway-specific additions and annexes to Building Regulation Lists A and B and List C of the DIBt (edition 2005/1), 2006 edition |
| 2006-557-NL | Regeling bodemkwaliteit | Preliminary draft of the Soil Quality Regulation |
| 2007-90-DK | Bygningsreglement 2008 | Building Regulations 07 |
| 2007-372-FIN | Rakennusten sisäilmasto ja ilmanvaihto Määräykset ja ohjeet 2007 | Indoor Climate and Ventilation in Buildings Regulations and Guidelines 2007 |
| 2007-653-A | Richtlinie für Recycling-Baustoffe aus Hochbau-Restmassen | Guidelines for recycled building materials from building demolition waste |
| 2007-9016-N | Utkast til nytt kapittel om forbrukerprodukter i produktforskriften: Miljøgifter i forbrukerprodukter | Draft of new chapter concerning consumer products in the Norwegian Product Regulations: Hazardous substances in consumer products |
| 2008-243-S | Föreskrifter om ändring i Kemikalieinspektionens föreskrifter (KIFS 2008:2) om kemiska produkter och biotekniska organismer | Regulations concerning changes to the regulations of the Swedish Chemicals Agency (KIFS 2008:2) on chemical products and biotechnical organisms |
| 2008/273/FIN | Ympäristöministeriön asetus rakennusten sisäilmastosta ja ilmanvaihdosta, Suomen rakentamismääräyskokoelma, D2, määräykset ja ohjeet | Directive of the Ministry of the Environment on indoor climate and ventilation of buildings, the National Code of Building Regulations of Finland, D2, Regulations and Guidelines |
| 2008-321-GR | Ανώτατο όριο φορμαλδεΰδης για τα έπιπλα, διακοσμητικά αντικείμενα και πρώτες ύλες αυτών σύνθετης συγκολλημένης ξυλείας – Μετανάστευση ορισμένων στοιχείων από παιδικά έπιπλα - Φορέας Επίπλου | Draft ministerial decision – maximum formaldehyde levels for furniture, decorative objects and laminated strand lumber (LSLs) used as raw materials in such – migration of certain elements from children’s furniture – Furniture Agency |
| 2008-565-NL | Regeling van de Minister van Volkshuisvesting, Ruimtelijke Ordening en Milieubeheer en de Staatssecretaris van Verkeer en Waterstaat houdende wijziging van de Regeling bodemkwaliteit | Regulation of the Minister for Housing, Spatial Planning and the Environment and the State Secretary for Transport, Public Works and Water Management amending the Soil Quality Regulation |
| 2009-34-F | Arrêté relatif aux conditions de mise sur le marché des produits de construction et de décoration contenant des substances cancérigènes, mutagènes ou reprotoxiques de catégorie 1 ou 2. | Order on the conditions on placing construction and decoration products containing carcinogenic, mutagenic or reprotoxic substances of category 1 or 2 on the market. |
| 2009-104-F | Projet d'arrêté relatif aux conditions de mise sur le marché des produits de construction et de décoration contenant des substances cancérigènes, mutagènes ou reprotoxiques de catégorie 1 ou 2 | Order relating to the conditions for marketing construction and decorative products containing carcinogens, mutagens or reprotoxic substances in category 1 or 2. |
| 2009-167-D | Entwurf der Grundsätze zur gesundheitlichen Bewertung von Bauprodukten in Innenräumen, Stand Oktober 2008 | Draft Provisions for health assessment of construction products in indoor environments, version of October 2008 |

http://ec.europa.eu/enterprise/construction/cpd-ds/

Including the following obligations: EU or EEA suppliers of articles which contain substances on the Candidate List in a concentration above 0.1% (w/w) have to provide sufficient information, available to them, to their customers or upon requests, to a consumer within 45 days of the receipt of the request. This information must ensure safe use of the article and as minimum contain the name of the substance.

Substitutes COUNCIL DIRECTIVE of 27 July 1976 (76/769/EEC) on the approximation of the laws, regulations and administrative provisions of the Member States relating to restrictions on the marketing and use of certain dangerous substances and preparations from 1st June 2009

The new rules for classification and labelling substituting the current classifications under Directive 67/548/EEC and 1999/45/EC apply in respect of substances from 1 December 2010 and in respect of mixtures from 1 June 2015.

Including the Consolidated list of existing active substances for which a decision of non-inclusion into Annex I or IA of Directive 98/8/EC has been adopted and which may no longer be placed on the market (Commission Decision 2007/565/EC of 14 August 2007, Commission Decision 2007/598/EC of 27 August 2007, Commission Decision 2008/681/EC of 28 July 2008, Commission Decision 2008/809/EC of 14 October 2008)

Directive 2000/60/EC of the European Parliament and of the Council of 23 October 2000 establishing a framework for Community action in the field of water policy

repealed by the Directive 2006/118/EC with effect from 22 December 2013

The term “materials” always refers to primary and secondary materials.

The term “test methods” refers to the whole assessment scheme of specific construction products. Therefore, it should include:

- sampling plan, sampling and transport and storage of a construction product (see mandate M/366, annex 2 on sampling),

- test specimen preparation,

- digestion/extraction

- analysis on the regulated dangerous substances content

- overall measurement/test report (see mandate M366)

Definitions of VVOC/VOC/SVOC:

a) by the World Health Organization (WHO)

VVOC:	boiling point is in the range from < 0°C to (50°C to 100°C)

VOC:	boiling point is in the range from (50°C to 100°C) to (240°C to 260°C)

SVOC:	boiling point is in the range from (240°C to 260°C) to (380°C to 400°C)

b) by European Collaborative Action Report No 18

VVOC:	All compounds which, in a capillary column coated with 100% dimethylpolysiloxane, are eluted before n-hexane are classed as VVOCs

VOC:	All volatile organic compounds which, in a capillary column coated with 100% dimethylpolysiloxane, are eluted with a retention range between n-hexane and n-hexadecane are classed as VOCs

SVOC:	All semi-volatile organic compounds which, in a capillary column coated with 100% dimethylpolysiloxane, are eluted with a retention range between n-hexadecane and n-docosane are classed as SVOCs

c) by some national regulations and RL 2004/42/EG

VOC:	Any organic compound having an initial boiling point less than or equal to 250°C measured at a standard pressure of 101,3 kPa

Substances classified carcinogenic, mutagenic or toxic for reproduction, CMR substances, are listed in Annex VI Part 3 of the Regulation (EC) Nr. 1272/2008.

The Polish Regulation from 1996 is included on legal grounds although it is not notified as it derives from the time before Poland joined the EU and the regulation is not in contradiction with community law.

Included on the Candidate List of Substances of Very High Concern for Authorisation and recommended for inclusion in the list of substances subject to authorisation under REACH (see: http://echa.europa.eu/consultations/authorisation/draft_recommendations_en.asp).